DISK UTILITY Version 7.5-D - For CP/M-86 ver 1.1 & CCP/M-86 ver 3.1-4.1

Originally by Ward Christensen for CP/M-80

To invoke "DU", simply enter "DU" at the CP/M-86 prompt.

C>DU

An initial command string may optionally be placed as an operand of

the original "DU" command, i.e.:

C>DU M;X

for example, if you want to only map the disk, and then exit.

Once "DU" is running, it expects single-letter commands much like "DDT".

For ease of use, multiple commands may be placed on one line, separated

by "!".  In addition, a given command or string of commands may be repeated

-- either indefinitely (until ^C is pressed) or a given number of times.

To avoid an accidental ^C from dropping out of "DU", only the explicit "X"

or ESC command will exit "DU".

COMMANDS: (Numeric values 'n' are decimal, 'x' are hex)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

+[n]   step in [n] sectors;

-[n]   step out [n] sectors

#      print disk parameters for curr drive.

=xxx   search for ASCII xxx from curr sector.

Caution: upper/lower case matters.

Use <xx> for hex:

To find "IN AL,0C0h" use: =<E4><C0>     or

"(tab)H,0(CR)(LF)" use: =<9>H,0<D><A>

<      save current sector into mem. buff.

>      restore saved sector

?      give help

A[ff,tt] ASCII dump

C      Change:

CHaddr,byte,byte... (hex)

or   CAaddr,data...  (Ascii)

<xx> Allowed for imbedded hex.

or   CHfrom-thru,byte  e.g. ch0-7f,e5

or   CAfrom-thru,byte

D[ff,tt] Dump (hex+ASCII)

Fn.t   Find file

Gnn    CP/M Allocation Group nn

H[ff,tt]       hex dump

L      Log in drive

Lx     Log in drive x

M[nn]  Map [from group nn]

N      New disk

P      Toggle printer switch

Q      Quiet mode (no msgs)

R      Read current sector

Snn    Sector nn

Tnn    Track nn

Unn    Set User nn for Find command

V[nn]  View [nn] ASCII sectors

W      Write current sector

X      Exit program

Z[nn]  Sleep [nn tenths]

/[nn]  Repeat [nn (decimal) times]

Cancel a function with ESC or Ctl-C.

Suspend output with S or Ctl-S.

Separate commands with "!".

Example: g0

+!d!z#20!/

would step in, dump, sleep 2 sec,

and repeat until control-c typed.

All "nn" usage except "/", "T", and "S" are HEX.  Use #nn for decimal.