#Subject:#   Project status report

#From:#     Bob Groppo

#Date:#     Sept. 20, 1982

#PROJECTS COMPLETED#

# New tech test#

a) This test is for placement of technicians and is currently

in use

# CMI M10 and M16 Review#

a) written review is available

# MOS customer service rackmount [Grayco]#

a) MOS 1.4 fixed problem (system still not returned to

Grayco for reasons I don't understand)

# MOS customer service table tops (Compumetrics)#

a) written report is available

# Decision 1 line filter #

a) Corcom 3EP1 spec'd - this requires new wiring of cabinet

and retesting to FCC specs

#MOS 1.4 for marketing on M16#

# MOS Masters and update#

a) intergrated into production

# CMI Restore under MOS #

a) Load constants resets the STEP line

#MPZ80 EPROMs for CMI M16 Drives (fix bugs)#

a) restore FFFFh tracks

b) Load Constants includes step reset

c) Image map matches actual map when in monitor

d) Monitor now enables the EN INT bit on Mult I/O

# Advanced Micro Digital switch settings #

a) preliminary release as Apnote 3

# Universal cooling#

a) appears to require a minimum of 90 CFM given

a 10 degree rasie from ambient and 300 watt power

disappation

# 8" HDC/DMA conversion kits#

a) PLL adjustment for the HDC/DMA manual appendix

b) preliminary documentation available (needs artwork)

.pa�#MULTI/O clock problem#

a) MOS 1.4 solved this problem with the Mult I/O although

a hardware reason for why the fix works is not apparent.

My suspicions are the 2 volt supply we apply to the CMOS

clock.

#�Cablin�/ Jumperin� Decision system� fo� standa�d printer�#�

a) prelimnary release as apnote 5

# 6 Meg HDCA #

a) the state machine needs work to support this-

merely swapping clocks does not work reliably.

# Change rt3@mult and ramtest 3 #

a) will be on the new test diskettes (E.3)

(input 03 changed to a NOP in the beginning)

#C Itoh evaluation#

a) written report available

# Miniscribe buffered seek evaluation #

a) written report available

# Qume 8" drive evaluation#

a) written report will soon be available

b) one 8 inch full size consistantly ate diskettes

c) slimline 8 inch ate one Dysan diskette and no other

diskettes yet although still under test

# HDCA Bad spot mapping#

a) verified new BAD program functional

b) errors reported from vendors need to be documented yet

in the appropriate manuals

.pa�               #PROJECTS STILL IN PROGRESS#

# E.3 Review #

(awaiting new release from software )

# MPZ80 Rev 3 CPU #

a) awaiting 82S153 parts from SIG (equations completed)

b) substantial artwork changes to fix current bugs

NMI, ERROR, pSYNC driver etc.

c) new clock circuitry (possibly Zilog)

d) 8 Mhz spec coming from Zilog

# Big switcher#

a) preliminary tests - first switcher smoked

but am now awaiting new supplies with newer specs

# FCC - Tests Required#

a) Micro Decision System checkout with various serial cables

b) Decision 1 checkout with new filter and serial cables

# 256K RAM board #

a) first 4 boards assembled - waiting for correct delay lines

these boards will need debugging and checkout/burnin

b) trace cuts and jumpers need documentation

c) evaluate RAM chips (packages and speeds)

qualified so far: T.I., Hitatchi and Mitsubishi

d) parity option checkout

e) documentation

# G & G Engineering#

a) Visit location to find what their problems are

with the HD/DMA systems and HDCA systems

# Centronics to daisy converter#

a) Software drivers are specified

b) Board pinout is established and will be protoyped

when parts become available

c) awaiting connectors from T & B Ansley

d) provided above is acceptable, layout the board

# Change uDecision seek test for random read/write#

a) all done except for an assembly language random routine

# 220V Micro Decision Supply#

a) Stancor stepdown transformer

1) Mounting documentation

2) electrical connection documentation

3) European plug issue needs resolution

# Tech training sessions# (course outline & materials)

a) preliminary outline for MPZ80 and HDC/DMA completed

.pa�

#PROJECTS NOT IN PROGRESS#

# Motherboard tests# (MPZ80 EPROM)

a) Current EPROM needs to check interrupts, daisy port and

clock

# PAL and EPROM programming solutions# for production

a) parts need to be programmed without master proms

(ideally through an automated proceedure using a

computer and serial port to down load from diskette)

# Morrow Transformer specs#

a) all power transformers and supplies need

checking / specs / schematics

# MULT I/O Battery Backup#

# Ken's printer # -jumpered for 9600 baud operation

# DJ/DMA Test procedures checkout#

# MOS does not stop on I/O errors during inits#

# MOS UPM erase problem #

a) this happens after erasing 13 files from UPM

# MOS Drive validation#

a) needs to use the badmap from FORMATMW

# Production board testing#

a) testing multiple boards with one bus

1) requires roms pals and proms

b) complete test documentation

1) disk tests

2) memory tests

3) interrupt tests

.pa�         #PROJECTS  BEGINNING WITHIN NEXT TWO WEEKS#

# Qume 5" drive evaluation#

Scheduled test of 20 slimline double sided

Scheduled long term test of 2 evaluation units

#Cartridge disk evaluation#

Extensive checkout of 5 inch 5 meg removable media

# Miniscribe 1006 evaluation#

Extensive checkout of two drives (fire sale)

.pa�            #ONGOING DOCUMENTATION PROJECTS#

#All Manuals#

Theory of operations for each board (2 weeks per manual)

#Manuals needing technical update:#

HDCA Rev 4 (disk format and error description)

HD/DMA     (Load constants, interrupts  and drive select

PLL adjustments in appendix)

DJ/DMA     (technical input, PLL check in appendix)

DJ2D/B     (re-write)

I/O        (rewrite with I/O charts)

256K Memory (manual to be written, technical input)

DISCUS Installation

#Applications Notes#

Release of Ap notes 2 - 5

#Miscellaneous Documentation#

Board Parts lists (substitution list)

Written test proceedures for production

Bipolar Prom cross reference chart

################################################