DOCUMENTATION FOR TDD v.2

Ronald E. Friedrich, c/o John of Beverley Chapel for the Deaf,

7611 Park Lane, Dallas, Texas 75225. (Correspondence is welcome.)

DESCRIPTION:  Use your Kaypro II as a Telecommuncations Device

for the Deaf (Baudot TTY).

FEATURES:

-TDD send/receive

-Autodialer (stores up to 26 phone numbers).

-Manual dialer (Dial from the blue keypad).

-Send pre-typed message of ANY length.

-Save your TDD conversations on disk file.

HOOKUP

To use this program you need an acoustic coupler designed for

Deaf TTYs.  If you do not already own one, ask a deaf friend for

the name of your local deaf TTY agent.  Second hand couplers may

also be available in some areas.  Other sources of couplers are:

Phone-TTY, Inc.                    Ultratec

14-25 Plaza Rd.                    P.O. Box 4062

Fair Lawn, NJ 07410                Madison, WI 53711

NOTE:  Computer modems will NOT work for deaf TTY purposes.

The coupler must be interfaced to the Kaypro parallel printer

port.  I have provided schematics for two interface circuits

depending on the type of coupler you use.  The interface circuits

require no extra power supply.

The interface includes an optional 12-volt relay for the autodial

feature of the program.  The "normally closed" contacts of the

relay should be wired in series with the telephone line.  The

relay BREAKS the phone circuit, simulating rotary dial pulses.

The dialer will work on all rotary or touch tone phone systems.

---------------------------------------------------------

This Kaypro-to-coupler interface presumes common ground on coupler jacks,

+voltage on pins of both "keyboard" and "printer" jacks.  All transistors

(marked 'EBC') are NPN (2N2222 or equiv) silicon switching transistors.

Kaypro II +----\/\/\/\---------+

Parallel  |     100K           |            to coupler "Keyboard" jack

Port (J2) |          B   /C----+------------------o------------  (+v)

pin 2-----+--\/\/\/\---|<                         o-------

(TDD send)      1K       \E                       |gnd

|

gnd                   to coupler

pin 11-----------C\   B                     "magnet/printer" jack

(TDD rcv)          >|------\/\/\/\---------+-----o------------ (+12v)

E/          1K            |     o-------

|gnd               1N914  |     |

200 ohm   Diode  |    gnd

gnd------)|--+--\/\/\/\----|<----+

100mf  |

|  n---->c  relay contacts (nc) in series with

|     :  o      phone line

+--+---+ :

|      )||

|      )||  12v relay

|      )||

+--|<--+  diode (1N4000 series)

|

1K      B   /C

pin 4-------/\/\/\/-----|<

(dial)                    \E

|

pins 19-30---gnd          gnd

(gnd)

===================================================================

This Kaypro-to-coupler interface presumes common ground on coupler jacks,

+voltage on pin of "keyboard" jack and -12 volts on pin of "printer" jack.

Unless otherwise marked, all transistors (labeled 'EBC') are NPN (2N2222 or

equiv) silicon switching transistors, resistors are 1000-ohms, and diodes

are 1N914.

100K

Kaypro II   +------\/\/\/\----+

Parallel    |                 |             to coupler "Keyboard" jack

Port (J2)   |        B   /C---+------------------o------------  (+v)

pin 2------++-\/\/\/\--|<                        o-------

(TDD send) |             \E                      |

|              |                     gnd

|             gnd

|                          Relay contacts (nc) in series

|                              c<----n  with phone line

|                              o  :

|                                 : +--------+

|                             12v ||(        |

+--------------------+       relay||(        |

|            ||(        |

B  /E              +--->|---+ Diode (1N4000

B  /C--\/\/\/\--|<PNP         B  /C        |    series)

pin 4-\/\/\/--|<               \C--\/\/\/\--|<          |

(dial)          \E                            \E        |

|                             |  100mf |         Diode

gnd                            +---)|---+--\/\/\/--|<--gnd

|          200 ohms

|

Diode   50K ohm   Diode      | "magnet/printer" jack

gnd------>|--+--\/\/\/\---->|--------+-o------------ (-12v)

|                         o-------

|                         |

pin 11-----------------+                        gnd

(TDD rcv)

pins 19-30-----gnd

(gnd)

*************************************************************

Next you need to connect your interface box...

1) To the acoustic coupler:

Insert the two plugs in the appropriate jacks in back of the coupler.

2) To the phone line:  (optional)

This connection is for the dialer only.  It is not necessary

to make this hookup to use the computer as a TTY.

Your phone wiring looks like this:

______

O    O   Telephone

/    \

------

||

||      wall terminal block

||     /

|+----o-------------------------------------

+-----o-------------------------------------

red & green wires (ignore black & yellow)

The computer dialer should be connected in series with the phone.

This means you must disconnect ONE of the phone wires (it does

not matter which: either red or green), and connect the two sides

of the open circuit to the normally closed contacts of the relay

in your interface box.  (See below.)  You can use light-gauge

speaker wire for that hook up.

______

O    O

/    \

------

||

||      wall terminal block

||     /

|+----o------------------------------------

+-----o +----------------------------------

| |  \disconnect this wire from the phone.

| |

__|_|__

| O O |

|_____|interface box

This connection may be semi-permanent.  It will not interfere

with the normal operation of the telephone.

3) To the Computer:

The computer cable plugs into the parallel printer port in

back of the Kaypro.  Obviously, you must first disconnect your

printer.

The start-up procedure I recommend is:

a) Load the TDD program.

b) While that is going on, remove your printer plug and

instert the TDD interface plug.

c) Insert the phone into the coupler cradle.

**************************************************************

OPERATION

When you load TDD.COM into the computer it automatically starts

up in the TDD (send/receive) mode.  If the coupler is plugged in

and turned on, you will hear beeps as you type, and see what you

type displayed in capital letters.

The program runs just like a C-PHONE.  It automatically generates

a <line feed><carage return> at the end of each line.  You can

also manually send <CR><LF> by hitting the <RETURN> key.  And you

can manually send a bare <LF> by hitting either the <LF> key or

down-arrow.

<DEL>,<BACKSPACE>, and left-arrow all function as a TDD back

space.

You never need to worry about shifting between <FIGS> and <LTRS>

because that is all handled automatically.  However, if the shift

mechanism gets confused while receiving (as can happen with any

TDD), you can manually set <FIGS> by hitting the up-arrow, and

<LTRS> by hitting the right-arrow.

************************************************

When you hit ESC you will be presented a SPECIAL FUNCTIONS menu:

D> DIALER

When you select the DIALER function, you will be presented with

a record of names of people whose phone numbers you have stored

in memory.  The first time you do this you will get a blank menu

list.  You can load names and numbers by using the "Modify"

option, as indicated on the menu (the <DEL> key).  Be sure there

is at least 2-Kbytes of space on the TDD disk for the phone

number records.

When you dial a number, the computer will automatically go back

to the TDD mode when finished.  You can abort dialing by hitting

the <ESC> key.  It will momemtarily hang up the phone and take

you back to the dialer menu.

You can exit from the dialer menu to the TDD mode by hitting

<ESC>.  Or you can exit from the dialer menu to CP/M by hitting

CTRL-C.

The dialer program turns a relay on and off, simulating

the switching action of a rotary dial.

W> WRITE THE LAST 16K OF THE PRECEDING CONVERSATION TO DISK.

If you want to keep a record of your conversation, select this

function.  You will be asked to enter the name of the file you

want to write.  The program will write everything sent and

received up that point.  If the conversation was very long, only

the last 16K is saved.  When writing is complete, the program

will return to the TDD send/receive mode.

You may later review your conversation with either CP/M's "TYPE"

command (after exiting TDD.COM), or with your word processor.

SENDING A MESSAGE:

First prepare your message on a disk file using your word

processor program.  Don't worry about upper/lower case letters,

as the TDD program will convert everything to upper case.  It

will also convert non-alphanumeric symbols to standard TTY

"figs."   Otherwise, the TDD program will send your file exactly

as you typed it.  It will even convert any <TAB> characters to an

appropriate number of spaces  (only in the MESSAGE mode, not in

the regular TDD mode).   Save your message on file, exit the word

processor, load TDD.COM, hit ESC, and select menu option:

M>  MESSAGE FILE NAME

The computer will ask you to name the file you want to send.  For

the sake of time, do this function BEFORE you make your phone

call.  The message will not be sent yet.  Rather, you will be

taken back to the TDD mode when you hit return. Then when you are

ready to actually send the message, hit ESC, and select menu option:

S> SEND MESSAGE FILE

The computer will start sending your message.  To interrupt at

any time simply hit any key.

When the message is completed (or interrupted), you will be taken

back to the TDD mode.

WILL TDD.COM PRINT HARD COPY?   No. The parallel printer jack is

busy with the TDD send/receive and dialer operations.  It cannot

simultaniously handle printer data.  However, you can save your

conversation on disk file, exit TDD.COM, and print that file.

==================================================================

TECHNICAL DATA

For those who like to know such things.

Telecommunication Devices for the Deaf use the 5-bit Baudot code,

45.5 Baud. (Start/data bits = 22ms; stop bit 31ms.)  TDD acoustic

couplers (half-duplex asyncronous send/receive) frequencies: Mark

= 1400Hz, Space = 1800Hz.

Telephone dial pulse rate is 10 pulses per second.  Pulse shape:

58%-64% break(open).  Minimum interdigit time: 600ms.  [Off-hook

resistence: 200 ohms, max.]

There are two ways to handle the 45.5 baud in the Kaypro:

1) Modify the SIO baud rate clock.  Such an approach is used by

Jim Larson (Software Design, P.O. Box 2722, Boise, Idaho 83701)

for his Ham (RTTY) program.

2) Drive the coupler with bare data from the PIO, incorporating

carefully timed delay loops in the software.  This is the

approach I have chosen.  Ergo, no hardware modification is needed

to run this program.

NOTE: This program was written for a 2.5MHz CPU clock.  If your

Kaypro has been modified to run a faster clock, you will need to

change the delay loops via the FUNCTIONS menu (hit ESC and select

option 'L').  Suggestion: If you are running a 5Hz clock, double

the delay loop values.  Don't be afraid to play with it; you can

easily restore timing variables to original default values.

It is not necessary to use a commercially marketed coupler if you

want to build a TDD modem with available PLL and VCO chips.

The parallel printer jack carries the following data:

Pin 2 (ASCII bit 0):        TTY serial data OUT

Start bit: low

Stop bit:  high

Pin 4 (ASCII bit 2):        DIAL serial data OUT

Off-hook (make): low

On-hook (break): high

Pin 11 (handshake line):    TTY serial data IN

[system port bit 3]             Start bit: high

stop bit:  low

The obvious discrepancy in TTY I/O logic was chosen to simplify

interfacing with available couplers.

================================================================

Daryl,

Here is more on the design of the program, which I did not

include in the original DOC file for the sake of space.

The program initially shuts off the disk drive motors, sends a

"silence key click" command to the keyboard, and sets the

data-out bit high (to quiet the coupler while it...) sets up the

ASCII/BAUDOT conversion tables.  When that is done, it tests the

data-in bit to see if the coupler interface is connected (bit is

low).  If the port is open (bit is high), you will get a prompt

to connect the coupler.  Note that the program may give you the

"ready" prompt if you have a parallel printer attached to the

port and the printer is off.

The heart of the program is a loop which alternately tests the

keyboard and data-in bit.  If a signal in is detected (bit goes

high), then it goes into the receive routine.  The program

samples the bit at five timed intervals, converts those five bits

to single binary number, looks it up on the tdd-to-ascii

conversion table, prints it, and stores it in the "save" buffer.

If keyboard data is detected, the program checks for an ESC

character, which will call the special functions menu.

Otherwise, the character is looked up on the ASCII-to-TDD table

(the printable equivalent is also in the table; i.e. all capital

letters, tdd printable characters).  The individual bits of the

tdd character are fed sequencially to the data-out bit, with

timed delay loops.

Since we all know that one can usually type faster than 45.5

baud, a "type ahead" feature is included in the send routine.

The keyboard is sampled at the beginning of every bit

transmission, and the data is temporarily stuffed in the "key-in"

buffer.  During the transmission of the stop bit, the program

scans the key-in buffer, and transfers good data to 256-byte

type-ahead buffer, and prints it on the screen.

Before the program checks for received data again, it checks the

type-ahead buffer.  If there is untransmitted data there, it will

transmit the next waiting byte, still checking the keyboard, and

adding data to the type-ahead buffer, if necessary.  Once the

send routine has emptied the type-ahead buffer, then it will

sample received data.

Note that the type-ahead buffer is a loop buffer.  There are two

byte counters: BUFFIN and BUFFOUT.  BUFFIN - BUFFOUT = the number

of unsent bytes in the buffer.  Because both of these byte

counters are single byte variables, they will automatically loop

to zero after 255.

There is also a byte count variable for line length, which will

automatically generate a CR-LF when one is necessary.

Note that all my chatter before about port and bit assignments

relate to the fact that many of my commands are HARDWARE

specific.  In other words, I'm reading and setting specific bits

at specific chips as designated by the hardware port assignments

(as I got them from the hardware schematic), not by CP/M's BDOS

and BIOS memory locations.

#fer.  If there is untransmitted data there, it will

transmit the next waiting byte, still checkin