When the Advent TurboROM is installed on a Kaypro (with or without

ZCPR3) several new video features are enabled, as detailed in the

TurboROM manual, pp. 5-9 and 5-10.

The side-effects of one feature are not immediately obvious, so the

reasons for using the control sequences in Par. 5.6.1.4 are unclear, as

is the manner of implementing them.

The TurboROM default, which provides a 24-line scrolling window with a

locked 25th line (for clock or status display) has the unintended

side-effect of slowing down scrolling, whether through text, or large

directories.  It also trashes the bottom line display used by some game

programs (CLONE is a good example).

The file L25.SET is simply a hex string representing the video control

code <ESC> C 7.  Using PIP, the command PIP CON:=L25.SET will unlock the

25th line and speed up scrolling.  On my K10 it also cleans up the

scrolling nicely (much less cursor flashing & will-o'-the-wisp garbage

floating across the screen). Of course, this change is not very helpful

when you want a locked, real-time clock display line.

To switch back to the default locked 25th line, easiest is probably to

cold boot (removing the PIP command from any STARTUP alias--since I

don't use a clock, I included the unlock sequence in STARTUP for

convenience).  It's also possible to make an alternate L24.SET by using

DDT (or SPZ, or ...) to change location 0101h from 43h [C] to 42h [B].

There's probably a much easier way to handle this, and to access other

TurboROM features.  Perhaps some assembly-language maven will put

together a bells & whistles package.

CREDITS: Thanks to Derek McKay of Plu*Perfect Systems for patient

explanations of these arcane, and pointing to various possible

solutions.  TurboROM and its associated code is copyright by Advent

Products, Inc. and Plu*Perfect Systems.  The ziggurat of ZCPR3 is

copyright by Echelon, Inc.

Fred Lieberman (N7AX)

voice tel.  (408) 338-2914

######################################################################