The following is an update to Dr. Liddle's '83 Kaypro 256K RAMDRIVE

modification, which allows the fine mod to be made to the '84 machines.

There may be a better way of doing it, but this works, and requires no

mainboard mods - unless you have to put sockets under your RAM chips.

You'll need his K256BIOS.LBR, in addition to this file.

Configure RAMDRIVE.MAC for your machine. I use the Advent Turbo-Rom, and

operate on a 61.75K system. My address for DIRBUF is 0FEE5H and for

RAMBUF 0F5ADH. I set RAMDRIVE to load at 0F363H, which is above my

ZCPRII stack and EXTFCB. Set the PIO DATA and CONTROL equates to the

appropriate port - see the PIO mods below to determine port addresses.

Follow Dr. Liddle's procedures for removing the old RAM chips. Install

sockets, if necessary, and wire all pin #1's together on RAM sockets.

Test the machine thoroughly with the enclosed MT256.COM - which will

crash if your CP/M is bigger than 62.75K - but it tests each location

up to the point of crash (when it overwrites CP/M) for every possible

read/write value. An error will appear if any cell is bad.

Build the overlay in Figure 1 below. Please note the additional decoupling

cap added between 5v and 0v at one or more points on the overlay. Mount

the overlay behind the Monitor ROM, so that all wires are as short as

possible. Don't forget to bend out pin 24 from U-29 so that it doesn't go

into the socket!

If your machine is a 4/84 OR 10/84 WITH internal modem or real time clock,

skip to the piggyback mod. If no RTC or modem, then install a PIO in position

U-35 and tie PIO-1,2,3 from the overlay below to 27,28,29 on U-35. If U-27

is missing, install it also. Your ports for RAMDISK.MAC are 021H and 023H

for data and control, respectively. Set these equates near the end of

RAMDRIVE.MAC.

FOR 4/84's and 10/84's WITH internal modem and/or RTC, add another PIO now.

Piggy-back a new Z80-PIO chip carefully on top of the old one as follows:

Solder Pins 1-3, 5-6, 11, 19-20, 22-26, 34-40 to the existing

PIO chip (carefully!).

Attach NEW PIO pin 4 to U-27 pin 13. This designates the NEW PIO

as Ports 029H and 02BH for data and control.

Tie PIO 1,2,3 from the overlay to 27, 28, 29 of NEW PIO. You also

end up with a spare side for another parallel port if you like.

Now install new 256K chips in the RAM sockets. Wire the overlay, keeping

all wires, ESPECIALLY 16 MHZ as short as possible, and keep them separated

if possible.

Turn on your machine. It should boot and perform properly. Check memory

again with MT256, and check ALL hardware - if ANYTHING appears abnormal,

fix it first! Run RAMDRIVE and you should be able to log into the RAMDRIVE

(C or D) and run BD.COM on it to test those memory locations!!

The theory is simple: We need the MUX and RFSH signals that the '84's

lack. U6 buffers the 16 mhz clock signal, and inverts 1/RFSH from the

Z80. U5 generates a pulse from 1/MREQ and RFSH that allows U4 to

generate the missing MUX signal. I originally used side 2 of U4 to

generate a 1/CAS signal to replace the one from U-29, but that wasn't

necessary.

I'm running the RAMDRIVE on a Kaypro 4/84 that's fully-loaded. My BBS

uses both an external Hayes and the internal modem in a multi-user

configuration, a Kaypro Clock, and I've added an HDO controller and

Shugart 712 (stretched to 13 meg) HD. It's been online for about 12

hours as I write this, with no flaws evident. I managed to cut HD

access by 90% by placing my menus on RAMDISK!

I'd appreciate any comments/suggestions be left to me on GEnie (CWMCHAN)

or call AMY <> BBS at 904-725-7461 (300/1200) or 904-725-1226 (300).

This is kinda rushed, but hope it helps!!

\_TABLE I\_

Signal on                  Chip on

Board                      Kaypro '84

1/RFSH                     U43 pin 28

1/MREQ                     U43 pin 19

A7                         U43 pin 37

A15                        U43 pin 5

RA7                        U32 pin 9

RA8                        U32 pin 1

PIO 1                      NEW PIO Pin 27

PIO 2                      NEW PIO Pin 28

PIO 3                      NEW PIO Pin 29

16 MHZ                     Jnct of C84/R40/Y5/U29-39

Also, lift Pin 24 of U-29 to disable the existing A7/15 line to the

RAM chips.

\_FIGURE 1\_

+-----------------------------+

|             U1              |

|         +--------+          |

|  +------|    7   |- 0v      |

|  |     -|    4   |----------+

|  |     -|    L   |-

|  |     -|    S   |-

|  |  0v -|    3   |-

+--|------|    9   |- 0v

| +5v -|    3  1|----------+

|      +--------+          |

|                          |

|          U2              |

33R    |      +--------+          |

RA7  <-- / /---|------|        |- 0v      |

+------|    7   |-         |

+---------|    4   |-         |

|        -|    L   |-         |

|        -|    S   |-         |

|        -|    1   |-         |

|     0v -|    5   |- 0v      |          (RFSH)

|    +5v -|    8  1|----------o--------------------------+

|         +--------+                                     |

|                                                        |

|             U3                                         |

|         +--------+                                     |

+---------|        |- 0v        33R                      |

A7   >------------o--|    7   |------------ / /--------> RA8        |

\_|    4   |-----------------o------< PIO 2      |

PIO 1 >---------------|    L   |-< +5v >-/\/\/\_/ 4.7k               |

0v -|    S   |-----------------o------< PIO 3      |

A15   >---------------|    1   |-< +5v >-/\/\/\_/ 4.7k               |

0v -|    5   |------------------------+            |

+5v -|    3  1|- 0v                    |            |

+--------+                        |  (MUX)     |

|            |

U4                            |            |

+--------+                        |            |

-|    7   |- 0v                    |            |

-|    4   |-                       |            |

+5v -|    L   |------------------------+            |

-|    S   |- +5v                                |

+5v -|    7   |-----------------------------+       |

-|    4   |- +5v                        |       |

+5v -|       1|------------------------+    |       |

+--------+                        |    |       |

|    |       |

U5                            |    |       |

+--------+                        |    |       |

0v -|    7   |- 0v                    |    |       |

0v -|    4   |- 0v                    |    |       |

-|    L   |- 0v                    |    |       |

0v -|    S   |-                       |    |       |

0v -|    0   |-------------------+    |    |       |

-|    2   |------> 1/MREQ     |    |    |       |

+5v -|       1|------------------------+    |       |

+--------+                   |         |       |

0v <--| |--> +5v                                  |         |       |

U6                       |         |       |

0.1 mfd         +--------+                   |         |       |

50 v.         -|    7   |- 0v               |         |       |

0v -|    4   |-----------------------------+       |

-|    H   |------> 16MHz      |                 |

0v -|    C   |-                  |                 |

-|    T   |- 0v               |                 |

0v -|    0   |-------------------o-----------------+

+5v -|    4  1|------> 1/RFSH

+--------+

###################################################################################