The following VT100 escape sequences are performed by MacKermit 0.8(34) VT100

emulation:

Cursor Movement Commands:

Cursor Up                           ESC[PnA

Cursor Down                         ESC[PnB

Cursor Forward (right)              ESC[PnC

Cursor Backward (left)              ESC[PnD

Direct Cursor Addressing            ESC[Pl;PcH or ESC[Pl;Pcf

Index                               ESC D

New Line                            ESC E

Reverse Index                       ESC M

Save Cursor Attributes              ESC 7

Restore Cursor Attributes           ESC 8

Character Attributes:

ESC[Ps m

where Ps refers to a selective parameter as follows:

0 or none               All Attributes Off

1                       Bold on

4                       Underscore on

5                       Reverse Video on (VT100 uses Blink)

7                       Reverse Video on

Erasing:

From Cursor to End of Line          ESC[K or ESC[0K

From Beginning of Line to Cursor    ESC[1K

Entire Line containing Cursor       ESC[2K

From Cursor to End of Screen        ESC[J or ESC[0J

From Beginning of Screen to Cursor  ESC[1J

Entire Screen                       ESC[2J

Character Sets:

The G0 and G1 character sets are designated as follows:

Character Set               G0 designator   G1 designator

United Kingdom (UK)         ESC(A           ESC)A

United States (USASCII)     ESC(B           ESC)B

Special graphics characters ESC(0           ESC)0

and line drawing set

[Alternate Set] (USASCII)   ESC(1           ESC)1

[Alternate Set] (USASCII)   ESC(2           ESC)2

and special graphics

Scrolling Region:

ESC[Pt;Pb r

Pt is the number of the line of the top of the scrolling region; Pb is the

number of the bottom line of the scroling region and must be greater than

Pt.

Tab Stops:

Set tab at current position                 ESC H

Clear tab as current position               ESC[g or ESC[0g

Clear all tabs                              ESC[3g

Modes:

To Set                  To Reset

Mode Name           Mode      Sequence      Mode      Sequence

Insert/Replace      Insert    ESC[4h        Replace   ESC[4l

LineFeed/NewLine    NewLine   ESC[20h       LineFeed  ESC[20l

Screen Mode         Reverse   ESC[?5h       Normal    ESC[?5l

Origin Mode         Relative  ESC[?6h       Absolute  ESC[?6l

Wraparound          On        ESC[?7h       Off       ESC[?7l

Auto Repeat         On        ESC[?8h       Off       ESC[?8l

Reports:

Cursor Position Report

Invoked by          ESC[6n

Response            ESC[Pl;PcR

Status Report

Invoked by          ESC[5n

Response            ESC[0n  (terminal OK)

What Are You?

Invoked by          ESC[c or ESC[0c

Response            ESC[?1;2c  (VT100 with AVO, Advanced Video Option)

Printer Status Report

Invoked by          ESC[?15n

Response            ESC[?13n  (no printer)

Editing:

Insertion Mode: Enter       ESC[4h

Exit (Replacement Mode)     ESC[4l

Delete Character            ESC[PnP

Delete Line                 ESC[PnM

Insert Line                 ESC[PnL

The following sequences are NOT performed by MacKermit VT100 emulation:

Line Size:

Change this line to double-height top half          ESC#3

Change this line to double-height bottom half       ESC#4

Change this line to single-width single-height      ESC#5

Change this line to double-width single-height      ESC#6

Character Attributes:

ESC[5m      uses reverse video, not blinking video

Programmable LEDS:

ESC[Ps q    set LED on or off

Modes:

To Set                  To Reset

Mode Name           Mode      Sequence      Mode      Sequence

Cursor Key Mode     Application ESC[?1h     Cursor    ESC[?1l

ANSI/VT52 Mode      ANSI        N/A         VT52      ESC[?2l

Column Mode         132 Col     ESC[?3h     80 Col    ESC[?3l

Scrolling Mode      Smooth      ESC[?4h     Jump      ESC[?4l

Interlace           On          ESC[?9h     Off       ESC[?9l

Keypad Mode         Application ESC=        Numeric   ESC[>

Print Termination   Form Feed   ESC[18h     None      ESC[18l

Print Extent        Full Page   ESC[19h     Window    ESC[19l

Print Control Sequences:

Auto Print Mode: Enter              ESC[?5i

Exit               ESC[?4i

Printer Controller mode: Enter      ESC[5i

Exit       ESC[4i

Print Line                          ESC[?1i

Print Page                          ESC[i

Report Terminal Parameters:

ESC[P1 n

Request terminal parameters

Reset:

ESC c

Confidence Tests:

ESC#8

ESC[2;Ps y

All VT52 mode commands:

Cursor Up                           ESC A

Cursor Down                         ESC B

Cursor Right                        ESC C

Cursor Left                         ESC D

Select Special Graphics             ESC F

Select ASCII Characters             ESC G

Cursor to Home                      ESC H

Reverse Line Feed                   ESC I

Erase to End of Screen              ESC J

Erase to End of Line                ESC K

Direct Cursor Address               ESC Y l c

Identify                            ESC Z           response:  ESC / Z

Enter Alternate Keypad Mode         ESC =

Exit Alternate Keypad Mode          ESC >

Enter ANSI mode                     ESC <

Enter Printer Controller mode       ESC W

Exit Printer Controller Mode        ESC X

Enter Auto Print Mode               ESC ?

Exit Auto Print Mode                ESC _

Print Line                          ESC V

Print Page                          ESC ]

#########################