Stephen Graham Pratt

Clare College

Emulating a DEC PDP-8/e

A Project for the Diploma in Computer Science 1995

Candidate:	Stephen Graham Pratt, Clare College

Title:	Emulating a DEC PDP-8/e

Examination:	Diploma in Computer Science 1995

Word Count (approx.):	10,000

Project Originator: 	Dr LC Paulson

Project Supervisor: 	Dr LC Paulson

Original Aim:

The aim of this project was to create a functionally correct emulator for the basic model DEC PDP-8/e computer (but with the full 32 kilowords of memory), with a graphical user interface to emulate the appearance of the real machine. As proof of the correctness of the implementation it was hoped that the emulator would run DEC's diagnostic test programs and the interpreter for FOCAL (a language supplied by DEC). It was also hoped that the emulator would run at a speed comparable with that of the original PDP-8/e.

Work Completed:

A working emulator with a graphical user interface was produced. It successfully ran DEC's diagnostic tests about 24 times slower than the original computer. The FOCAL interpreter also ran correctly: both responding to input and executing a simple program at a perfectly reasonable speed.

Special Difficulties:

The actual PDP-8/e hardware, for which the diagnostic programs and the FOCAL interpreter were designed, did not conform precisely to the description in DEC's handbook. The discrepancies were sufficient to cause these programs to fail until the faults were found and corrected.

The machine's interrupt system was intricate and sensitive to subtle errors in the implementation. These also caused failure until the faults were corrected.

Table of Contents

µChapter 1:  Introduction	1

Chapter 2:  Preparation	4

Summary	4

2.1  Understanding the PDP-8/e	4

2.2  Thread Structure	5

2.3  Information Structure	6

2.4  Graphical User Interface	7

2.5  Planning for Evaluation	8

2.6  Insurance Against Accidents	8

Chapter 3:  Implementation	9

Summary	9

3.1  Library	9

3.2  Main Processor Cycle	10

3.3  User Interface	11

3.4  Control Thread	13

3.5  Instructions	13

3.5.1  Memory Reference Instructions	13

3.5.2  OPR Instructions	14

3.5.3  IOT Instructions	15

3.6  Interrupts	16

3.7  Extended Memory	18

3.8  Teletype and Paper Tape Reader	18

3.9  Removing Diagnostic Messages	20

Chapter 4:  Evaluation	21

Summary	21

4.1  Verification	21

4.1.1  Library	22

4.1.2  Main Processor Cycle and User Interface	22

4.1.3  Memory Reference Instructions	23

4.1.4  OPR Instructions	23

4.1.5  Interrupt System	24

4.1.6  Extended Memory Instructions	26

4.1.7  Teletype and Paper Tape Devices	26

4.1.8  DEC's Diagnostic Tests	27

4.1.9  Running u8-388 and FOCAL	27

4.2  Performance Testing	28

4.2.1  Diagnostic Messages	28

4.2.2  Register Accesses	29

4.2.3  Memory Accesses	29

4.2.4  Interrupts	29

4.2.5  Separate Threads	30

4.2.6  Best Performance	30

Chapter 5:  Conclusions	31

Bibliography	32

Appendix 1:  DEC's Diagnostic Tests	33

Appendix 2:  Instruction Formats	34

A2.1  Memory Reference Instructions	34

A2.1.1  Indirect Addressing	34

A2.1.2  Page Specification	34

A2.2  IOT Instructions	35

A2.3  OPR Instructions	35

Appendix 3:  Initialisation File	37

Appendix 4:  Interrupts	38

Appendix 5:  User Interface Commands	39

Appendix 6:  Paper Tape Formats	40

A6.1  RIM Format	40

A6.2  BIN Format	41

Appendix 7:  Removing the Diagnostics	42

Project Proposal	44

# Chapter 1:  Introduction

The PDP-8/e was a digital computer with a 4 kiloword core memory which could be upgraded to 32 kilowords. The control panel of the computer had two rows of indicator lights. The top row displayed the current address. The information displayed by the second row depended on the setting of the display selector switch. There were also twelve switches making up the switch register which was used in conjunction with the 'Address Load' and 'Deposit' keys to load data into memory. A fuller description of the keys on the front panel is given in appendix 5. The other facilities for input and output on the basic machine were a teletype (keyboard and printer) and a paper tape reader and punch.

The PDP-8/e was the most popular of the PDP-8 family of computers, which was introduced in 1965 and 'has been described as the Model T of the computer industry, because it was the first computer to be mass-produced at a cost that just about anyone could afford.' This popularity was also due to the many expansion options which were developed by DEC, including a magnetic disc system and an oscilloscope display, which, along with the software provided, helped to make the PDP-8 family one of the most versatile and widely used computers of its time. One of the software packages was the interpreted language FOCAL (FOrmula CALculator) which was supplied with the PDP-8 family by DEC. This was similar in style and power to contemporary versions of BASIC. It was 'used by students, engineers and scientists in solving virtually any mathematical problem, and much more.' There was also an assembler called PAL-8 which converted listings of machine instructions to files in BIN format which were then loaded as described below.

DEC also provided diagnostic test programs which could be run on the machine to verify that it was functioning correctly.

The usual method of reading a program into the basic PDP-8/e was longwinded. Firstly, the seventeen instruction RIM loader, which was capable of reading tapes punched in RIM format, was loaded using the keys on the front panel. This was then used to load the more sophisticated BIN loader which recognised the BIN format in which most PDP-8 tapes were punched. Finally, the BIN loader was used to read in the program required.

The PDP-8/e could either be run in continuous mode or single step mode. In continuous mode, which was entered by pressing the 'Continue' key, instructions were executed in sequence until either a halt (HLT) instruction was encountered or the 'Halt' key was depressed on the front panel. In single step mode, instructions were executed one at a time in response to depression of the 'Single Step' key.

The PDP-8/e was a single accumulator (AC) machine, with additional temporary storage in the multiplier quotient register (MQ). There was also a flags register which included a one bit link flag. This performed the same function as the carry on a modern machine.

All machine instructions were one 12-bit word in length. The top three bits of the instruction specified the operation code (opcode): hence there were eight basic instruction types. A brief summary is given below.

Opcodes 0-5 were memory reference instructions with the low nine bits specifying the memory location Y. These instructions were:

AND Y	logical AND between AC and Y, storing result in AC

TAD Y	add Y to AC using two's complement arithmetic

ISZ Y	increment Y and skip the next instruction if Y becomes zero

DCA Y	deposit AC at Y and clear AC

JMS Y	jump to subroutine at Y

JMP Y	jump to Y

The PDP-8/e had no hardware stack. The JMS instruction stored the address to which to return (i.e. the current value of the program counter) at Y and the actual code of the subroutine starts at Y+1. This method makes re-entrant subroutines difficult to write.

Opcode 6 indicated an input/output transfer (IOT) instruction: the rest of the bits specify the device to be addressed and a device-specific command.

Opcode 7 indicated an operate (OPR) instruction. These manipulated the AC, MQ and flag register contents (e.g. CMA (CoMplement Accumulator) and RTL (Rotate Two Left)) and provided conditional execution via instructions which will skip the next instruction (which may be a JMP instruction) if tests on the AC and flag registers succeed.

A program interrupt facility was also available: this needed to be implemented in the emulator for DEC's diagnostic tests and FOCAL to run correctly.

A few PDP-8 computers are still in use and occasional appeals for hardware, software and technical assistance for the family are posted on the Usenet news group alt.sys.pdp8.

The most significant previous work in the area which I found was a fully functional emulator written in C by Douglas Jones of the University of Iowa. This was not used as a major resource, as it was decided to implement the new emulator in Modula-3 using separate threads of execution. It was, however, used as a reference against which to compare the results of tests of the new emulator and when resolving a few of the more tricky problems encountered in the implementation.

Note: most PDP-8 documentation uses octal as its primary base, and that convention is followed in this report.

# Chapter 2:  Preparation

## Summary

Jones' emulator was compiled and tested. It was then decided exactly what should be implemented in the new emulator and the data structures required to represent its internal state were described. The separate asynchronous threads of execution required were defined. Modula-3 was chosen as the language in which to implement the emulator as it provides a well supported facility for such threads. The user interface design tool formsedit was investigated. Various features were specified to aid evaluation of the system. A system of regular backups was defined.

## 2.1  Understanding the PDP-8/e

Several files were downloaded from the World Wide Web. These were: the source for Jones' emulator, a cross-assembler for the PAL-8 assembly language used by the PDP-8 family, a program in BIN format called u8-388 (which, when given a year, produces a calendar for it), and a copy of the FOCAL interpreter.

A version of Jones' emulator with a graphical user interface similar to the front panel of the real machine was compiled. DEC's diagnostic test programs were run on this emulator to demonstrate that it is implemented correctly: it ran the second test program about three times slower than the original machine. The FOCAL interpreter and the calendar program were also successfully run on this emulator.

Running programs on the emulator gave an opportunity to get a feel for how the PDP-8/e worked and how it was used. Having done this it was decided to implement the basic machine (but with 32 kilowords of memory). It was decided that there probably would not be time to implement any optional extras, so these were not considered further at this stage.

DEC's books on the PDP-8/e itself and on how to program it were read. These were the main references used throughout the project. Both the source code of Jones' emulator and the Usenet news group alt.sys.pdp8 were used to clear up points where the description in these books was insufficiently precise.

## 2.2  Thread Structure

The inherent asynchrony of a computer system suggests the use of several separate threads of execution to implement an emulator. The threads used were:

1.	control thread: responsible for initialising the emulator, starting all the other threads, and tidying up at the end.

2.	main processor cycle: reads instructions from memory and implements them, also handles interrupt requests.

3.	user interface: accepts commands from the user, which may be issued while the processor is running.

4.	character reader: attempts to read characters from input files when requested by the main processor cycle; raises an interrupt (if appropriate) when a character is ready.

The separate threads enable the processor thread to run more quickly, without constantly needing to check for input from the user interface and from files. Modula-3 was chosen as the language to use as this provides good support for separate execution threads. It was therefore necessary to build the emulator from scratch because no other emulators written in Modula-3 have been found.

The use of separate threads does slow down the emulator but they made program design easier.

## 2.3  Information Structure

The emulator is represented as a set of objects as shown in figure 1.

µ §

The 'Core Status' object records the current status of the emulator. The registers used are: accumulator, program counter, link register, instruction register, multiplier quotient, memory buffer, switch register, SW line register, instruction field, data field, instruction field buffer, instruction field save register and data field save register. The last two in this list are used by the interrupt system to save the current values of the field registers.

The inter-thread signals are:

·	a signal from the processor informing the user interface that a cycle has been completed

·	a signal from the user interface requesting the processor to execute one or more instructions

·	a signal which can be raised by any thread to request that the control thread terminate the program

The execution status consists of two flags indicating:

·	whether the emulator is currently executing an instruction

·	whether it is in continuous mode

The system bus is represented as an object containing a separate object to represent each device attached to the bus. The bus object has a command method which is invoked by the processor cycle to implement all IOT instructions. This in turn invokes the command method of the appropriate device to implement the instruction. Each device is responsible for implementing a specific group of IOT instructions.

## 2.4  Graphical User Interface

Modula-3 provides support for graphical displays on X terminals via a system called FormsVBT. Graphical displays are implemented as an object hierarchy. To simplify the task of designing a display, the formsedit tool is provided. This uses a Lisp-like language to specify the display, and allows the designer to try out specifications as they are typed in. The FormsVBT system provides procedures to create displays from files containing such specifications.

The use of macros to describe repeated non-standard objects (e.g. indicator lights and push switches) in this language was successfully investigated.

The attachment of callback procedures to objects in the display in order to implement the functions of keys on the front panel was also investigated. It was found that these procedures are required to have a standard signature. This makes it impossible to pass an object representing the status of the emulator directly into the procedure. Therefore, a pointer to this object must be set up in the module containing the procedure when the program is initialised.

## 2.5  Planning for Evaluation

The Unix Revision Control System (RCS) was used to store each revision of each source code file, so that results of any test could be accurately reproduced.

The profiling system which is part of SRC Modula-3 was investigated. This records the number of times each line and procedure in the source code is executed. This shows which the most heavily used sections of code are, and hence the areas which need optimisation. Unfortunately there is an error in the SunOS5 issue of Modula-3 which prevents profiling information from being produced. It was therefore necessary to obtain an account on the Computing Laboratory Unix system to give access to SunOS4 machines (where the bug is not present) on which profiling could be performed.

The following facilities were included in the emulator to aid evaluation:

1.	The ability to produce diagnostic messages while running. These messages were used to pinpoint errors and to verify parts of the program. The messages were divided into different types and the types to be produced during a given run may be specified in the initialisation file.

2.	The ability to load and save core memory images. Programs to inspect these images and to produce them from files in BIN format were also written. BIN format is the form in which most PDP-8 paper tapes were produced, and in which programs are available from the World Wide Web. The ability to convert from BIN format and then load directly into the emulator makes loading programs much easier than the method used on the original machine, which is described in chapter 1.

3.	The ability to start the emulator automatically at a predefined address when the program is run, and to stop it after a specified number of instructions. This allowed tests to be run over long periods of time without a user interface. It also allowed tests to be run at night when the system was less heavily loaded.

## 2.6  Insurance Against Accidents

To prevent accidents or system crashes from causing a disastrous loss of data, all source code, documentation and test result files were combined together twice weekly using the Unix tools 'tar' and 'gzip' and stored on the Pelican archive system. RCS is also useful in this respect: if a source file is accidentally deleted, only the changes made since the file was last checked into RCS are lost.

# Chapter 3:  Implementation

## Summary

The emulator was implemented in sections as described in this chapter. After each section had been written it was tested as described in chapter 4. The main stages of implementation were as follows:

·	A library of foundation modules was built

·	The main processor cycle (with dummy procedures to implement instructions) and the keyboard user interface were implemented. These were implemented simultaneously so that they could be used to test each other.

·	Instructions (and all the associated hardware emulation) were then added along with the interrupt system.

·	The whole emulator was tested for correctness.

·	The graphical user interface was added.

As a result of the evaluation described in chapter 4, changes were then made to the implementation to improve the performance of the emulator. This included a program (written using lex) to reversibly comment out all calls to the procedure generating diagnostic messages. Other improvements are described in chapter 4.

## 3.1  Library

The foundations on which to build the emulator were placed in a library. These include the objects in the 'Core Status' section of figure 1 as well as modules to:

·	return values of parameters specified in an initialisation file

·	produce diagnostic messages

There are also several points at which the emulator needs to be able to manipulate individual bits within a binary number. Procedures to convert between binary numbers and equivalent arrays of BOOLEANs were therefore included in the library so that individual bits could be accessed easily when required.

The registers module supplies an object type representing a single register. The size of a register is specified when the object is instantiated. Read, write and increment methods are provided. These allow the register value to be accessible only through object methods, which can then ensure that only valid values can be written to a register. The implementation of registers was changed when it was found that the emulator is slowed down significantly by the method described here.

Reading from, writing to, and incrementing memory locations were originally implemented as methods of the memory object so that the interface would be transparent to the way in which the contents of memory were actually stored. This was later changed in an attempt to speed up all memory accesses, and reads from memory in particular. The memory module includes the ability to load and save images of the core memory of the emulator as specified in section 2.5. This uses the built in Modula-3 interface called Pickle which allows any reference type (including any object) to be stored in a Unix file.

To ensure correct signalling between threads an object type was created which stores a signal if there is no thread waiting for it. (Modula-3 condition variables discard any signal for which no thread is waiting.) The next thread to wait on a signal via that object will detect the signal (and clear it) and will not block. An example of where this is necessary is the interlock between the processor thread and the user interface when running in single step mode. The user interface thread sends a signal requesting the processor to execute a cycle, and then waits for a signal from the processor that the cycle has been completed before proceeding. If either signal is issued before the target thread is ready for it, and if the signal is then lost, as would occur with standard condition variables, then deadlock will occur as each thread is waiting for a signal from the other.

## 3.2  Main Processor Cycle

The main processor cycle implemented by the processor thread is a loop which runs until either stopped by the user via a 'Halt' command from the user interface or because the emulator has executed the number of instructions required.

The cycle proceeds as follows:

·	If interrupts are enabled the thread will check whether any are pending. If there are, it will invoke the interrupt handler as described in appendix 4.

·	The instruction pointed to by the program counter is loaded and, if it is a memory reference instruction, the operand is decoded to determine the address to which it refers.

·	The program counter is incremented.

·	The instruction is executed.

·	The processor signals to the user interface that it has finished a cycle.

·	If appropriate (i.e. if in single step mode or if halted while executing this instruction) the processor then waits for a signal from the user interface before executing the next instruction.

## 3.3  User Interface

The information structure for both user interfaces is as shown in figure 2.

µ §

The user interface must have access to the 'Core Status' object to be able to display parts of its contents, and to deposit values into core memory. Access to the 'System Bus' object is required because the user interface is responsible for attaching files to input and output devices by invoking connect and disconnect methods of these devices which are contained within the bus object.

The keyboard user interface commands were designed with the following considerations in mind:

·	close emulation of the front panel keys

·	speed of use

·	ease of implementation

It was not required that the user interface commands be particularly clear or intuitive as they were only intended to be used while developing the system.

The procedures to implement the commands were, where possible, written so that the same procedures could be used by the graphical user interface. This meant that their signatures had to conform to the signature required of a callback procedure by FormsVBT.

Commands given to the user interface are searched for in an array containing each command paired with a pointer to the procedure to be invoked to implement it. The commands requiring further parameters are required to read these from the input for themselves.

The user interface is responsible for displaying status information comprising:

·	the current address

·	whether the emulator is running

·	other information depending on the display mode.

The keyboard user interface prints this information after each command is completed. The graphical user interface updates the indicator lights with this information after each depression of a key on the front panel and whenever the user interface thread is scheduled while the emulator is running in continuous mode. This ensures that the display is kept reasonably up to date without sacrificing speed by insisting on an update during each processor cycle.

The status update is achieved by the procedure pointed to in the user interface object. This pointer is set up on initialisation depending upon which user interface (if any) is in use. The shared user interface code is then able to use this pointer to invoke the appropriate update procedure.

The picture between pages 12 and 13 of this report is the final graphical user interface. The image was captured when FOCAL had just been initialised and was ready to accept a program. The top section is a slightly adjusted version of the front panel of the original machine. The rows of squares are the indicator lights. The diamonds are a set of radio buttons representing the display selector switch: the area alongside this gives details of what the lights mean for each setting of the selector. Below this is the bank of switches whose functions are described in appendix 5. These are implemented as a type of button so that clicking the mouse on them has the same effect as pressing the switch on the original machine.

The next section of the display (below the row of switches) contains fields in which to specify files to be connected to the paper tape device along with buttons to perform the actions of connection and disconnection. Below this there is a box in which certain messages relating to attempts to connect these files are displayed. At the bottom of the display is a scrollable window which displays the teletype output. Characters typed when the cursor is over this region are treated as teletype input.

## 3.4  Control Thread

The control thread initialises the data structures and threads which make up the emulator. Much of the information to do this is obtained from the initialisation file. Defaults are used for any parameter not given in the initialisation file, or if no initialisation file is specified.

The most complex task is setting up the user interface. This is because several settings depend on whether the graphical or keyboard user interface (or none) is to be used. A pointer to the appropriate status update procedure must be set up. If the graphical user interface is required, connections must be made between the object on the display emulating the teletype and the device object which implements the teletype instructions (and hence needs access to a source and a sink for teletype characters).

The initialisation file may also specify Unix files to be connected to the paper tape device (and to the teletype device if the graphical user interface is not being used): these connections are made by the control thread.

Having initialised everything, the control thread starts the other threads running and waits for one of them to signal that the program should terminate. (This may be because the user has requested termination, because the specified maximum number of instructions has been executed, or due to an error condition.) The control thread then signals to all the other threads that they should finish and waits for them to do so. (It is therefore necessary for all threads to check for this signal frequently and to finish quickly when the signal is detected. The signal is implemented using Modula-3's system of alerts.)

Finally the current state of core memory may be saved to a file before the program exits.

## 3.5  Instructions

### 3.5.1  Memory Reference Instructions

The memory reference instructions (opcodes 0-5) were implemented as an array of procedures indexed by opcode. An alteration which might speed up the emulator would be to incorporate the code of these procedures as a CASE statement in the main processor loop to remove the overhead of a procedure call. The problem which was encountered when implementing these instructions is described in section 4.1.3.

### 3.5.2  OPR Instructions

Two alternative methods were used to implement OPR instructions. The first was simpler to implement and compiled more quickly. The implementation of the second method is more complicated but is derived from the code for the first. It took significantly longer to compile as the source code module which implements the instructions is over 8,000 lines long, but was expected to execute more quickly for reason given below.

The first method identifies which group the instruction is in and then invokes the procedure which executes instructions in that group. This procedure examines the rest of the bits in the instruction and executes code for each of the (possibly multiple) actions specified by these bits. (There is a prescribed order in which the functions in a group are to be executed and the bits are examined and the functions executed in this order.)

The second method is implemented as an array containing a pointer to a separate procedure for each possible bit combination in any of the three groups. The operand of the OPR instruction is then used as a single nine bit number to index into this array. These procedures were generated by a program which decodes each possible instruction and writes the required commands to a Modula-3 source file. This program was developed from the code used for the first method above by altering it to write commands to a file rather than executing them directly. The second method was expected to execute faster as the decoding of the instruction has already been performed at compile time.

Clarity of implementation, rather than efficiency, was aimed for. Improvements would have been possible within the implementation of individual functions but these would only have had a marginal improvement on overall speed. The exception to this would be programs making particularly heavy use of the individual functions whose implementation was improved.

To make the choice of method independent from the rest of the program, both methods were implemented as modules exporting the OPR interface which contains a single procedure. The sole purpose of this procedure is to pass information from the processor cycle into the code implementing the instructions. The method required can then be chosen by specifying a switch at compile time. This switch is tested in the m3makefile where it is used to decide which module (and hence which method) to link into the program. This technique produces an overhead of at least one extra procedure call per OPR instruction.

A possible way to speed up execution (of the first method) would be to remove this intermediate interface and to decode the instruction to discover its group in the main cycle. The appropriate procedure to implement that instruction group could then be called directly from the processor cycle. Switching to use the second method of implementing OPR instructions would then require alteration of the source code of the processor cycle to index into the array of precompiled procedures rather than calling the appropriate instruction group's procedure.

The instructions which were particularly awkward to implement were the rotation instructions. These treat the link and the AC together as a single thirteen bit register. The instructions specify a rotation which may be by either one or two bits and either to the left or to the right. Bits which 'fall off' one end are replaced at the other. Each of these instructions involves a significant amount of manipulation of individual bits to achieve the correct result.

### 3.5.3  IOT Instructions

IOT instructions are all implemented as commands to devices on the system bus. The command method of the bus object is invoked, and the operand of the instruction and a pointer to the core status object are passed to it as parameters. Access to the 'Core Status' object is needed as its contents may be altered by the code implementing the instruction. The appropriate device is chosen in accordance with the device selector portion of the instruction.

The command portion of the instruction and the core status pointer are then passed on to the command method of the device object which in turn implements the instruction. Some of the device objects actually implement commands for more than one value of the device selection bits, so these bits are also passed to the device object.

There is one exception to this: the CAF (Clear All Flags) instruction. This clears the AC and link registers, and then resets interrupts and all the devices attached to the bus. To perform this last task it is necessary to invoke each device object's reset method. If the CAF command were passed on to a device object according to its device selector bits, it would then be awkward to arrange for it to have access to each device on the bus so that it could reset them. Instead, CAF is handled in the command method of the bus itself, where the bus object is accessible, and through it all the device objects.

## 3.6  Interrupts

For details of the PDP-8/e interrupt system, see appendix 4. It is implemented in two sections: the object containing the current state of interrupts and the device implementing the interrupt related instructions.

Several different unsuccessful attempts were made at an object to correctly implement the current state of the interrupt system. Finally, an object with the following methods was produced:

1.	determine whether interrupts are enabled

2.	determine whether interrupts have been enabled and the one cycle delay has elapsed.

3.	determine whether any interrupt requests are pending

4.	inform the object that another processor cycle has completed, so that it can keep track of the one cycle delay

5.	inform the object that interrupts have been enabled or disabled

6.	inhibit and uninhibit interrupts

7.	raise and cancel interrupt requests for individual devices

8.	reset the interrupt system (used when CAF is issued)

Each device may have at most one interrupt pending at any one time. So that the system can keep track of which devices do have interrupts pending, each device has a unique identifier (allocated at initialisation) by which it identifies itself to the interrupt status object when raising and cancelling interrupts.

The use of execution profiling shows that methods 2 and 4 above are called during each processor cycle. To speed up the execution of these methods, the structure of the interrupt status object was changed so that a minimum number of variable manipulations is performed in each cycle. The fields required were also made directly accessible by the processor cycle to remove the overhead of making a procedure call to invoke an object method. The final implementation was:

1.	a counter which is incremented at the end of each processor cycle

2.	a cycle number indicating whether interrupts are enabled. If the counter is not less than this number then interrupts are active.

3.	a BOOLEAN flag to indicate whether interrupts are inhibited

4.	a set of flags indicating which devices' interrupts have been requested and not cancelled

5.	a counter giving the total number of interrupts currently pending. This allows the processor to quickly check whether any requests are pending.

To enable interrupts, the cycle number (2) is set to two more than the current value of the counter (1). The counter is incremented at the end of each cycle and interrupts are taken at the start of a cycle, so an interrupt (if there is one pending) will first be taken after a delay of one cycle, as required.

To disable interrupts the cycle number is set to the largest representable number, which it is assumed that the counter will always be less than (but see below).

A side effect of this method is that the emulator is limited to executing a certain maximum number of instructions per run. This maximum is the largest number representable in a variable of type CARDINAL. The version of Modula-3 being used here uses 32 bit integers. As the type CARDINAL is defined as [0..LAST(INTEGER)], the largest CARDINAL is the same as the largest INTEGER: that is 231 - 1, which is about 2 x 109. An average instruction on the PDP-8/e took 2.6 ms, so this represents about ninety minutes' computation. If the count does reach this number then interrupts may be incorrectly enabled for one cycle, but then the emulator will fail, as the program will be terminated by the Modula-3 run-time system when an attempt is made to increment the counter beyond its upper limit.

Difficulties were encountered with the GTF, RTF and SKON instructions. This became apparent when DEC's diagnostic test programs failed.

SKON tests whether interrupts are enabled. If they are, it disables them and skips the next instruction. The problem with SKON was that it should skip the next instruction if interrupts are enabled irrespective of whether the one cycle delay has elapsed: i.e. the sequence ION SKON should cause the skip to be taken. I had originally assumed that this sequence should not cause the skip.

GTF and RTF are 'get flags' and 'restore flags' respectively. These perform transfers between the flags register and the accumulator. The actual PDP-8/e implements the GTF and RTF instructions in a different way to that described in the handbook. The differences are that the actual values of some bits are ignored in the transfers and assumed to be either always set or always clear. These small discrepancies were sufficient to cause the diagnostic tests to fail. The problems were corrected and the diagnostic programs then ran correctly.

## 3.7  Extended Memory

IOT instructions are provided to change the current instruction and data fields, and to read the current state of the field registers. The IF register is not changed immediately when the CIF (Change Instruction Field) instruction is issued. Instead the instruction buffer register (IB) is set to the new instruction field and interrupts are inhibited. When the next JMP or JMS instruction is encountered, interrupts are uninhibited and IF is set to the contents of IB. This allows transfers of execution between fields to be programmed easily by the sequence:

CIF field

JMP address

The field registers are saved when an interrupt is taken. There is an extended memory instruction to restore these values into the IB and DF registers which is used when returning from an interrupt.

The most complex part of implementing these instructions was ensuring that interrupts are correctly inhibited when a new value is placed in the IB register. It was also necessary to ensure that the JMP and JMS instructions correctly detect and deal with the inhibition, by loading the contents of IB into IF and uninhibiting interrupts.

## 3.8  Teletype and Paper Tape Reader

These were probably the most difficult parts of the emulator to implement as they require interaction between separate threads and the raising of interrupt requests. The implementations of the two devices have a lot in common and so are implemented using the object structure shown in figure 3.

The implementation of a character writer is straightforward. It is assumed that an output file is always ready to accept a character, so all that is needed is to write the character to the file. (It would be possible for a disc error to occur to prevent the write from taking place: in this case the writer will stall and never become ready again.)

The character reader object is actually a closure for a separate thread. (There is one thread each for the tape and teletype objects.) This waits until requested to fetch a character: it then blocks until a character is ready, and returns it in a one character buffer. If interrupts for the device are enabled, an interrupt is requested. The extra thread is necessary because characters can be typed at the teletype at any time, and an interrupt should be raised as soon as a new character is ready.

Commands are provided in the keyboard user interface to connect files to these devices. These invoke the connect and disconnect methods of the device objects as appropriate. When using the graphical user interface with integrated teletype, the teletype is connected on initialisation to file-like objects which act as channels between the teletype window on the interface and the teletype device object. Fields are provided on the graphical user interface in which to specify files for the paper tape device, as described in section 3.3.

µ §

When FOCAL was first run, standard input and standard output were used for the teletype. Unix buffers input from the console until a carriage return is entered, so that input is presented to a process a line at a time. This caused characters to be presented to FOCAL faster than its interrupt handler could deal with them. (The original I/O hardware was slow, so FOCAL had plenty of processing time available between characters.) The initial solution to this problem was to force the character reader thread to wait for a fixed length of time before attempting to read the next character. This allowed time for FOCAL to process each character before being presented with the next. When the teletype was incorporated into the graphical user interface this problem was less acute as the teletype window presents characters to the reader thread individually as they are typed. Combined with the improvements in speed described in sections 4.2.1 - 4.2.4, this prevented FOCAL from crashing.

## 3.9  Removing Diagnostic Messages

It was found that the procedure which is called most frequently is the one which optionally produces diagnostic messages. (The procedure was called Diag.Message.) Even when no messages are actually produced there is a procedure call overhead, as the decision whether to produce a message is made within the procedure. This overhead slows the emulator by a factor of at least 10.

A program was therefore written to reversibly comment out these calls from the source code, along with a complementary program to reinstate them. This ensures that the diagnostics are still available, at the expense of a recompilation.

The programs were written using the Unix tool lex. The token recognition part of the program to comment out calls is given in appendix 7; the program to reinstate the calls is almost identical but with the reverse function. When commenting out calls, the string 'Diag.Message' is replaced with '(*Diag.Message' and the next occurrence of ');' is replaced with ');*)'.

The main function of the program (which is omitted from the appendix) reads a list of file names from a supplied input file, and executes the yylex function on each one to perform the replacement.

# Chapter 4:  Evaluation

## Summary

The project was evaluated in two stages.

Each module of the program was verified as it was developed to ensure that it functioned correctly. This was done using separate Modula-3 test harnesses for the library modules, and by running short PAL-8 assembler programs on the emulator for most of the rest of the modules. The whole emulator was then verified by running DEC's diagnostic tests.

When all the functions required had been implemented, the performance of the emulator was assessed. This was measured in terms of speed of execution, using DEC's second diagnostic test program as a benchmark. Execution profiling was used to find the sections of the program which were executed most frequently. The code in these sections was then optimised for speed of execution. A reasonable speed was achieved: this was sufficient, for example, to run FOCAL interactively without having to wait too long for a response.

## 4.1  Verification

Each module was tested when it was first developed. When significant changes were made during later development the tests were repeated to ensure that the module still functioned as required.

Most of the verification tests involved running the emulator in the state of development which it was in at the time. Status and diagnostic outputs were examined to determine the results of the tests.

Many of the tests were in the form of short programs written in PAL-8 assembler, which were run by the emulator. The version of the PAL-8 assembler which was obtained from the World Wide Web did not recognise all the mnemonics required to write test programs. (For example, it did not recognise the GTF and RTF instructions.) Extra entries were added to the assembler's symbol table to correct this. The programs were assembled into BIN format using the cross-assembler, and then run on Jones' emulator to ensure that assumptions made about their results were correct. They were then converted from BIN format into memory images to be loaded directly into my emulator. The results from running the programs were compared with those obtained from Jones' emulator.

Several references are made below to 'DEC's diagnostic tests running correctly.' Where this is in respect of corrections to a particular instruction this means that the tests no longer failed in the vicinity of that instruction.

### 4.1.1  Library

Each library module was verified by test harness programs written in Modula-3. The memory module tests ensured that a saved memory image was an accurate representation of the state of memory. (This was tested by saving an image and reloading it, and checking that the reloaded contents were the same as those saved.) The registers module was tested to ensure that only valid values could be written into a register.

### 4.1.2  Main Processor Cycle and User Interface

The processor cycle and keyboard user interface were tested simultaneously using tests which were sequences of user interface commands. Core memory was loaded with appropriate values using the 'Address Load' and 'Deposit' commands. These values were then used to test the rest of the commands. Each command to be tested was issued, and the resultant state of the emulator was examined to verify that the command had caused the correct action to be performed. Invalid commands were correctly rejected by the emulator. The graphical user interface was similarly tested by checking that clicking the mouse on each key had the desired effect.

The emulator was presented with memory reference instructions representing all possible addressing modes (including autoindexing) to check that the address was correctly calculated from the operand of an instruction.

The response of the emulator to a simulated interrupt request was also checked. If interrupts are enabled, it correctly stores the program counter and transfers control to location 0001 in field 0. The other actions to be taken on processing an interrupt request were checked when interrupts were fully implemented.

### 4.1.3  Memory Reference Instructions

Each of these was tested by writing a simple PAL-8 program to execute the instruction. For AND, ISZ and DCA this was simply a case of executing the instruction and examining the state of the accumulator and memory before and afterwards.

With TAD it was also necessary to inspect the state of the link register. Initially it was found that the program did not handle the this correctly. The implementation was corrected so that the link register is inverted if the sum of AC and memory exceeds 7777 (octal). In other words, link and AC together are treated as a thirteen bit register. Overflow from this larger register is correctly ignored.

The JMP and JMS instructions were tested using a program containing a simple subroutine. This tests that the writing of the return address and the transfer of control are correct, but does not test the treatment of interrupts or the changing of the IF register. These were tested as part of the extended memory system tests.

### 4.1.4  OPR Instructions

Identical tests were used for both interpreted and compiled versions of the OPR instructions. The tests described are not intended to be exhaustive as it would be prohibitively time-consuming to check that all 512 possible microcoded combinations of functions are correct. Instead, each individual function was tested and the DEC diagnostic tests were relied upon to highlight any errors arising from combining functions.

The test for group 1 instructions was simply a program which was executed in single step mode. The AC and link registers were examined after each instruction to verify that the results were correct. The rotate instructions were found to be incorrect because the direction of rotation was the opposite to that required. This was easily fixed.

An error not detected until DEC's diagnostic tests were run was that IAC (Increment Accumulator) should carry into the link register if the AC is incremented from 7777 (octal) to 0000. No indication of this is given in DEC's specification, but when the carry had been implemented, the diagnostic tests ran correctly.

The testing of group 2 instructions fell into two parts. The instructions which do not cause conditional skips were tested in the same way as the group 1 instructions: no problems were found here.

To test the conditional instructions, a program of the following form was used. The program was divided into several groups of instructions, each group being of the following format:

CLA	clear accumulator

SNA	skip if AC is non-zero

SZA	skip if AC is zero

HLT	halt

This tests that SNA will not skip, and that SZA will skip, if the accumulator is zero. To complete the tests of these instructions, their action under the opposite condition must be tested, so the following group is also needed:

IAC	increment accumulator (so it is no longer zero)

SZA

SNA

HLT

Groups like this were placed in sequence in a program to test each of the conditional skip instructions in group 2. If any of the skip instructions had been incorrectly coded, then the program would have halted at the end of the appropriate group. This did not happen, and the program ran to the end, demonstrating that the group 2 instructions were correctly implemented.

Testing group 3 instructions was (like group 1) performed by stepping through a simple program and examining the contents of AC and MQ after each step. (On the basic PDP-8/e group 3 instructions simply provide transfers between MQ and AC.)

### 4.1.5  Interrupt System

To test the object storing the current state of interrupts a Modula-3 test harness was written which simulates conditions to ensure that the object correctly senses requests to enable, disable, and inhibit interrupts, and correctly reports on the current state. Particular attention was paid to ensuring that the one cycle delay between enabling interrupts and allowing them to be taken was correctly implemented. The raising and cancellation of interrupts and reporting on the state of each device's interrupt flag was also checked.

The interrupt system instructions were tested by the following program:

SRQ	skip next instruction if interrupts are pending - there are none at this point

SKIP	unconditional skip - causes execution to proceed to IOF

*	HLT	halt - this occurs if the first instruction skipped in error

IOF	disable interrupts

SKON	if interrupts are enabled, skip next instruction and disable interrupts

SKIP	unconditional skip - in normal circumstances execution will proceed to ION

*	HLT	this should be skipped unless SKON incorrectly caused a skip

ION	enable interrupts after the next instruction

NOP	no operation

SKON

*	HLT

TFL	teletype instruction - side effect used here is to raise an interrupt.           NB interrupts are now disabled (by the SKON instruction) so the interrupt handler will not be invoked.

GTF	read system flags to check that the interrupt has been raised (by manually inspecting the contents of AC)

SRQ	skip if interrupt pending

*	HLT

ION	enable interrupts

NOP	after this instruction interrupts should be enabled, and there is an interrupt pending so it should be taken before the final instruction

HLT	correct execution will stop here having executed the interrupt handler

If interrupts are implemented correctly, this program should halt at the end having executed the interrupt handler once. Otherwise it should stop near the instruction in error. This program did not show up the problem with the SKON instruction which was detected by DEC's diagnostic tests.

The interrupt handler used contained instructions to ensure that the contents of IF and DF had been correctly saved on taking the interrupt. Interrupts must also have been correctly disabled before entering the handler: if they had not been, the emulator would have made repeated attempts to take the interrupt by jumping to location 0001 at the start of each cycle.

### 4.1.6  Extended Memory Instructions

A PAL-8 program was written to copy a sequence of words from field 0 to field 1. This checks that the data field can be changed correctly. The sequence of words constituted a program, which was then run by issuing the instructions:

CIF 1

JMP START

This checks that the new instruction field is correctly buffered until the JMP instruction is encountered. In fact, a slightly more complex sequence was used which involved simulating an interrupt between the two instructions above. This interrupt was not taken until immediately after the JMP instruction, demonstrating that interrupts are correctly inhibited by the CIF instruction and uninhibited by the JMP instruction.

The rest of the extended memory instructions were then checked by executing a program containing them in single step mode and checking the system state between steps.

### 4.1.7  Teletype and Paper Tape Devices

To test these devices, programs were written to read and write sequences of characters. For each action (read or write) and for each device, two programs were required. One of the programs used a 'busy waiting' loop to wait for the next character to be ready. (Or for the simulated hardware to be ready for the next character when writing.) The program tested that this technique could be used to read (or write) a string of characters. The other program tested the device's interrupt instructions by using an interrupt handler to perform the character processing: the main program does something else while waiting.

Success in all cases was denoted by the production of a string of characters either in an output file or in a previously empty memory block.

The biggest problem encountered was with reading characters. This is because the emulator must wait at least until the character reader thread is next scheduled before the next character can be read. This potentially wastes many instruction cycles: initially it was unclear that the interrupts were working, as a large number of cycles was executed before taking an interrupt. A possible solution is for the processor thread to block so that the character reader can request a character from the input. Care would be needed to ensure that the processor thread was unblocked and able to continue if the character reader thread blocked while waiting for the character.

Other minor problems were encountered and their solutions are described in section 3.8.

### 4.1.8  DEC's Diagnostic Tests

The emulator was set up to run overnight without a user interface and was run using the command:

nohup pdp8e -i test.init &

This combination ensures that the process is not stopped when the user logs out. Teletype output was redirected to a file. Success was indicated by a file of constantly increasing size containing either As or Bs.

Each test was only able to highlight one problem at a time as they are deterministic programs indicating faults by halting near the instruction which is in error. They did indicate small errors in the implementation of the SKON, RTF and GTF instructions as described in section 3.6, the error in the TAD instruction as described in section 4.1.3 and the error in the IAC instruction as described in 4.1.4.

Once the errors were corrected, the test programs ran continuously as expected.

### 4.1.9  Running u8-388 and FOCAL

The final verification test was to get the emulator to run some meaningful programs. The first was the calendar program (u8-388), which correctly produced a calendar for this year when presented with 1995 as its input.

Running FOCAL was more difficult, for the reason described in section 3.8. Once this problem had been ironed out, FOCAL ran correctly, though very slowly. The following program was run within the FOCAL interpreter, showing that it is possible to run a program within a program (FOCAL) within a program (the emulator)! The emulator was, of course, supported by many other programs in the form of the Unix operating system. The FOCAL program was:

1.1 TYPE "HELLO WORLD!"!

1.2 GOTO 1.1

The trailing ! on line 1.1 ensures that a carriage return is produced. To test FOCAL's mathematical functions, the command

TYPE FLOG(2)

was used. This correctly returned the result

=    0.6932

## 4.2  Performance Testing

At this stage, a correctly functioning emulator had been produced. However, its performance left a lot to be desired. The reason for needing to leave the diagnostic tests running overnight was that the second test program only produced a B approximately once every hour. (The first test program was no better.) This made the new emulator 720 times slower than the original machine, and 240 times slower than Jones' emulator.

The Modula-3 execution profiling utility was used to generate a breakdown of the number of times each user defined procedure was called while running the calendar program. The most frequently called procedures were:

| procedure | frequency |
| --- | --- |
| diagnostics generator | 663898 |
| read a value from a register | 317651 |
| write a value to a register | 200723 |
| read a value from memory | 156576 |
| increment a register value | 138053 |
| interrupt status object method 4 | 113305 |
| interrupt status object method 2 | 113305 |

These results motivated the changes described in sections 4.2.1 - 4.2.4 below. Some more general points on execution efficiency are then considered as described in sections 4.2.5 - 4.2.6.

### 4.2.1  Diagnostic Messages

By far the most commonly called procedure is the one which produces diagnostic messages. The program written to comment these calls out of the source code is described in section 3.9. The calls were not removed completely; another program was written to reinstate them for when they were needed for further debugging. The removal of diagnostic messages from the program sped it up by a factor of about 10.

### 4.2.2  Register Accesses

The presence of all of the methods of the register object type in the above list suggested that any optimisation performed here would be worthwhile. On reflection, it was realised that the register object type is unnecessary. It was therefore replaced by two Modula-3 ordinal types: Register_12 and Register_3, which were defined as the ranges [0..4095] and [0..7] respectively. It is still impossible to write invalid values to registers represented in this way. The only drawback is that overflow must be explicitly anticipated when incrementing the register. The removal of the procedure call overhead means that each register access now requires significantly less processing.

### 4.2.3  Memory Accesses

Memory reads, writes and increments were initially implemented as methods of the memory object. Reading from memory was the fourth most common procedure call in the table on page 28. To cut down the time taken to read from memory, these methods were removed and the contents of memory were instead made directly accessible. (The same drawback for incrementing a location applies as in the case for registers above.) Memory was also restructured as a set of eight pointers to arrays representing fields. This allowed a pointer to the current instruction and data fields to be stored alongside the current IF and DF values so that less processing is now required for each memory access. Since direct memory accesses are limited to two pages within the instruction field, this approach could be taken further by representing each field as an array of 32 pointers to pages. Pointers to the current and zero pages would then be sufficient to refer to all directly accessible locations. This could speed up memory access even further, though it would involve more processing as the page pointers would need to be updated each time execution moved into a new page.

### 4.2.4  Interrupts

The final improvement was in the implementation of the interrupt system, to prevent the two method procedure calls listed above from being made in each processor cycle. The improvements are described in section 3.6.

### 4.2.5  Separate Threads

It was also found that the emulator ran faster when no user interface was used. This is natural, as less processing is then needed. However, it does suggest that the use of multiple threads in general slows the program down, and that it would have run faster if it had been designed as a single thread, in the same way as Jones' emulator. As pointed out in section 2.2, separate threads were used to make program design easier.

### 4.2.6  Best Performance

The changes described in sections 4.2.1 - 4.2.4 above were implemented. The resultant emulator was run without a user interface on an otherwise unloaded Solaris 2 machine (ursa). This emulator, when running the second diagnostic test program, produced a B every two minutes. This is 24 times slower than the original machine and 8 times slower than Jones' emulator. This is a very large improvement on the unoptimised program.

Despite the figure given above, the emulator runs FOCAL at a perfectly acceptable speed when performing simple operations and I would guess that it runs at a speed comparable to BASIC on the microcomputers of ten years ago. (e.g. the BBC Microcomputer)

# Chapter 5:  Conclusions

As demonstrated in chapter 4, a successful PDP-8/e emulator was produced. This emulator functions correctly in that it passes DEC's diagnostic tests. It is also capable of running the FOCAL interpreter.

Though it runs FOCAL at a tolerable speed, the emulator runs the diagnostic tests about 24 times more slowly than the original machine. Given more time it would probably be possible to shave this figure to about 20. In the end though, the emulator is limited by the language and methods used to construct it.

Modula-3 is a strongly typed language with many built-in run-time checks. It also has a garbage collector. While these do help enforce the writing of robust code, they do introduce a large execution time overhead. This is a problem for programs such as an emulator where speed is a prime consideration. The use of separate threads also introduces a time overhead, though the raising of interrupts by devices as and when characters are ready would otherwise be difficult to implement.

To achieve an emulator closely approaching the speed of the real machine it would probably be necessary to start again from scratch. The emulator could then be planned speed as the primary consideration, sacrificing clarity of implementation where necessary. It would also be a good idea to use a language (such as C) which does not have significant run-time overheads.

A copy of the source for this project has been sent to Douglas Jones and should be available via his DEC PDP-8 index.

# Bibliography

Brown, M and Meehan, J:	The FormsVBT Reference Manual: version 2.2

(ã DEC 1992-3)

Harbison, S: Modula-3 (ã Prentice Hall 1992)

Jones, D:	DEC PDP-8 index: http://www.cs.uiowa.edu/~jones/pdp8/index.html

Jones, D:	PDP-8 Frequently Asked Questions:

http://www.cis.ohio-state.edu/hypertext/faq/usenet/dec-faq/pdp8/faq.html

PDP-8/e Small Computer Handbook 1971 (ã DEC 1970 (sic))

PDP-8 Handbook Series:	Volume 1: Introduction to Programming

(ã DEC 2nd ed. 1970)

Usenet news group: alt.sys.pdp8

# Appendix 1:  DEC's Diagnostic Tests

The tests used consisted of two programs which were obtained, along with minimal documentation, from Bob Supnik at DEC.

(Electronic mail address: supnik@human.enet.dec.com)

They are repetitive tests which are designed to be run for a long time to check that the hardware is functioning correctly. If they detect an error, they halt the machine at an address close to the instruction at fault. On the original PDP-8/e the tests rang the teletype bell every five seconds as a sign that they were still running. (On the versions used here this has been replaced by typing a letter every five seconds: A in the first test and B in the second).

The speed at which these tests ran on an emulator was used as a benchmark when assessing performance.

# Appendix 2:  Instruction Formats

The basic instruction format is shown in figure 4.

µ §

## A2.1  Memory Reference Instructions

The layout of a memory reference instruction (AND, TAD, ISZ, DCA, JMS or JMP) is as shown in figure 5.

Memory is divided into 8 fields each containing 32 pages of 128 words. The instruction field (IF) and data field (DF) registers specify the two currently accessible fields. The program counter (PC) always refers to an address in the current instruction field.

An instruction may directly address any location in the current page or in zero page of the instruction field, and may indirectly address any location in the data field (for AND, TAD, ISZ or DCA) or in the instruction field (for JMS or JMP).

### A2.1.1  Indirect Addressing

If the I bit is set, the address specified contains a pointer to the address to be referred to by the instruction. The instruction type determines whether this address is in the data or instruction field as specified above. In each field the locations from 0010 to 0017 (octal) are autoindexing. This means that when referred to as a pointer when indirect addressing is used, their value is incremented (and written back to the location) before it is used to specify the actual address to be referred to by the instruction.

### A2.1.2  Page Specification

If P is zero then the following 7 bits refer to an address in zero page of the instruction field, otherwise they refer to the page of the instruction.

## A2.2  IOT Instructions

These have the format as shown in figure 6.

µ §

The device selector bits indicate the device for which the instruction is intended. The three command bits specify the command, whose meaning depends on the individual device.

## A2.3  OPR Instructions

These are divided into three groups. Within each group, each bit specifies a different action to perform. If multiple function bits are set, then their actions are combined. There is a prescribed order in which the actions are combined. This is specified to make the combined instructions as useful as possible.

Group 1 has the format given in figure 7 and is primarily concerned with manipulating the contents of the AC and link registers. (The link is one of the bits of the flag register and is used as a carry flag.)

µ §

Group 2 has the format given in figure 8. The instructions in this group provide conditional execution and a few other miscellaneous functions. The conditional execution is provided in the form of instructions which cause the next instruction in memory to be skipped if conditions on the AC or link registers are met.

For example, to branch if AC is zero, the following code is used:

SNA	skip next instruction if AC is not zero

JMP address

...other code...	code to execute if AC is not zero

The layout of group 3 is shown in figure 9. The only functions in group 3 which are implemented on the basic machine are transfers between AC and MQ. The other functions in this group are only implemented if the extended arithmetic element is present.

# Appendix 3:  Initialisation File

On startup it is possible to specify an initialisation file for the emulator using the command:

pdp8e -i <filename>

If no file is specified or if the file cannot be found then default values are used.

The file is split into sections so that different parts of the emulator can easily identify the parameters intended for them. A new section is denoted by the string [<section name>] at the beginning of a line. A line beginning with a # is ignored.

This idea is taken from the form of initialisation files used by Microsoft Windows.

The sections are as follows:

[AutoExec]

InstructionLimit=<number>	limit instructions executed to this number

UserInterface=Graphical|Keyboard|None	specify user interface type

StartAddress=<octal>	start at this address in field 0

SwitchRegister=<octal>	start with switch register set to this value

AutoExec=TRUE|FALSE	start execution immediately and automatically

[CharReader]

Delay=<number>	number of seconds (as a LONGREAL) to wait

between accepting characters from the input

stream

[DiagSetup]

Stdout=TRUE|FALSE	display diagnostics on standard output

File=<filename>	write diagnostics to specified log file

[Diag]

<type>=TRUE|FALSE	turn on|off diagnostics messages of this type

[IOFiles]

Keyboard=<filename>	use this file as keyboard input

Teletype=<filename>	use this file for teleprinter output

Reader=<filename>	tape reader input file

Punch=<filename>	tape punch output file

[Memory]

InFile=<filename>	initialise memory from this file

OutFile=<filename>	dump memory image to this file on completion

# Appendix 4:  Interrupts

An interrupt request may be issued by any device on the system bus. Servicing an interrupt consists of storing the current value of the PC in location 0000 in field 0, disabling interrupts, saving the current values of the IF and DF registers and transferring execution to location 0001 in field 0. Interrupts are enabled by the ION and RTF instructions, but there is a one cycle delay before an interrupt will be taken. This allows interrupt handlers to return before re-enabling interrupts, and is necessary because subroutines are not re-entrant as there is no return address stack.

Interrupts may also be inhibited. This occurs when the IB register is loaded with a new instruction field, and prevents interrupts from being taken until the next JMP or JMS instruction when the new field is transferred to the IF register. This method of changing the instruction field makes it easier to write programs, as the field change is delayed until the next JMP or JMS instruction. Inhibiting is necessary so that a JMP or JMS instruction occurring in interrupt handling code does not upset the field registers, which would cause execution to return to the wrong field.

Each device has its own flag which is set when it issues an interrupt request, and provides an instruction which skips the next instruction if this flag is set. It is the responsibility of the interrupt handling code to ensure that the device clears its interrupt request and that it re-enables interrupts if appropriate.

# Appendix 5:  User Interface Commands

The commands recognised by the keyboard user interface are as follows. The front panel key to which a command is equivalent is given in parentheses.

AL: (ADDR LOAD)	set current address from the switch register

CL: (CLEAR)	clear AC, link, interrupts, I/O flags

CO {R|W} {K|T} <filename>*:	connect specified file to Reader | Writer of teletype(K) | Tape reader

DC {R|W} {K|T}:	disconnect file

DE: (DEP)	deposit value in switch register at the current address, and advance the address by one

DI <1-5>:	equivalent to settings of the selector on front panel: selects the extra information to display as described below.

EX: (EXAM)	examine value at current address, and advance the address by one

GO: (CONT)	start continuous mode execution

HA: (HALT)	stop continuous mode execution

HE:	print this information

QU:	stop emulator, tidy up and exit program

SR <xxxx>	set switch register

SS: (SING STEP)	execute single processor cycle

SW:	invert SW line

XL: (EXTD ADDR LOAD)	set the current instruction field from bits 6-8 of switch register and the current data field from bits 9-11 of switch register.

The implementation of the display modes for the second row of indicator lights differs slightly from the original hardware as many of the original signals have no equivalent on the emulator. The implementation used is as follows:

1: STATE:	bits 3-5:	opcode of current instruction

bit 8:	state of the SW line

2: STATUS:	bit 0:	link register

bit 2:	flag whether interrupts are pending

bit 3:	flag whether interrupts are inhibited

bit 4:	flag whether interrupts are enabled

bits 6-8:	instruction field register

bits 9-11:	data field register

3: AC:	contents of the accumulator

4: MD:	contents of the memory buffer register

5: MQ:	contents of the memory quotient register

# Appendix 6:  Paper Tape Formats

The two paper tape loading programs available with the PDP-8/e are Read-In Mode (RIM) and Binary (BIN). The RIM loader is a very simple program which can easily be loaded into the machine by hand from the front panel. This is then usually used to load the BIN loader which is able to read tapes which were punched using the more compact BIN format.

Paper tapes were eight bits (or channels) wide: the lower six bits were used to specify addresses and data, the seventh and eighth bits were used for control data. A hole in the tape denoted a binary one. A hole in the seventh tape channel indicated the first byte of an address. A hole in the eighth channel indicated leader or trailer tape. There was a ninth, small hole which was punched between the third and fourth channels in every row. This was the sprocket hole used to guide the tape through the reader.

## A6.1  RIM Format

Each data item is specified by two pairs of bytes on the paper tape. The first pair gives the address and the second pair specifies the value to be placed at that address.

The beginning of a RIM tape might be punched as follows:

| Channel: | 8 | 7 | 6 | 5 | 4 | S | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  | leader tape |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 0 | 1 | 0 | 0 | 0 | . | 0 | 1 | 0 |  | address: 0200 (octal) |
|  | 0 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 0 | 0 | 1 | 0 | 0 | . | 0 | 1 | 1 |  | value: 4317 (octal) |
|  | 0 | 0 | 0 | 0 | 1 | . | 1 | 1 | 1 |  |  |
|  | 0 | 1 | 0 | 0 | 0 | . | 0 | 1 | 0 |  | address: 0201 (octal) |
|  | 0 | 0 | 0 | 0 | 0 | . | 0 | 0 | 1 |  |  |
|  | 0 | 0 | 1 | 0 | 1 | . | 0 | 0 | 0 |  | value: 5026 (octal) |
|  | 0 | 0 | 0 | 1 | 0 | . | 1 | 1 | 0 |  |  |

## A6.2  BIN Format

BIN is more compact because it allows continuous blocks of values to be specified. If no address is specified it is assumed that the next value is to be placed in the next sequential memory location.

The beginning of an equivalent BIN tape might be:

| Channel: | 8 | 7 | 6 | 5 | 4 | S | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  | leader tape |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 1 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 0 | 1 | 0 | 0 | 0 | . | 0 | 1 | 0 |  | address: 0200 (octal) |
|  | 0 | 0 | 0 | 0 | 0 | . | 0 | 0 | 0 |  |  |
|  | 0 | 0 | 1 | 0 | 0 | . | 0 | 1 | 1 |  | value: 4317 (octal) |
|  | 0 | 0 | 0 | 0 | 1 | . | 1 | 1 | 1 |  |  |
|  | 0 | 0 | 1 | 0 | 1 | . | 0 | 0 | 0 |  | value: 5026 (octal) to be placed at |
|  | 0 | 0 | 0 | 1 | 0 | . | 1 | 1 | 0 |  | address 0201 (octal) |

Note that the second address does not need to be specified.

# Appendix 7:  Removing the Diagnostics

Below is the part of the lex script which recognises the appropriate tokens in the source file. The main program which deals with processing the source files has been omitted.

%{

/* various #include statements here */

#define FALSE 0

#define TRUE  1

/* flag whether currently processing a call to be commented out */

int commenting = FALSE;

/* code returned if an irretrievable error is encountered */

#define ERROR 1

%}

bra	\(

ket	\)

semicolon	\;

star	\*

opencomment	{bra}{star}

closecomment	{star}{ket}

procedure	Diag\.Message

endstatement	{ket}{semicolon}

%%

{opencomment}{procedure}	{	/* already commented Diag.Message */

/* echo to output */

fputs("(*Diag.Message", yyout);

}

{procedure}	{	/* not commented Diag.Message */

/* insert comment and note that */

/* we're processing a Diag.Message command */

if (commenting == FALSE) {

fputs("(*Diag.Message", yyout);

commenting = TRUE;

}

else {

fprintf(stderr, "error\n");

return ERROR;

}

}

{endstatement}{closecomment}	{	/* end of already commented call */

/* echo to output */

fputs(");*)", yyout);

}

{endstatement}	{ /* end of statement */

fputs(");", yyout);

/* if processing a call to be commented */

/* out, output closing comment and note */

/* that comment has finished */

if (commenting == TRUE) {

fputs("*)", yyout);

commenting = FALSE;

}

}

%%

int main (int argc, char **argv)

{

/* the main program follows here */

}

# Project Proposal

Emulating a DEC PDP-8/e

Suggested by Dr LC Paulson

Resources required: increase of CUS disc quota to 11Mb

Supervisor: Dr LC Paulson

Director of Studies: Dr LC Paulson

Overseers: Dr M Richards, Dr NE Wiseman

Description of Work

The project is to write a program in Modula-3 which will emulate the Digital Equipment Corporation PDP-8/e. The front panel will be implemented graphically using the Trestle user interface system for X Windows. The program will be compiled on a Solaris 2 machine (Ursa) and run under Unix.

The program will emulate the base model PDP-8/e, including a 4096 12-bit word main memory with teletype (via computer keyboard and X Windows window) and paper tape punch and reader (with the paper tapes emulated by standard files on the Unix filing system) as well as the front panel. Other options could be implemented if time permits.

Intermediate stages will be tested using test harnesses and diagnostic messages which are optionally activated at run-time. The finished emulator will be tested using existing test programs which are available on the World Wide Web.

Evidence of success would be supplied in the form of session records and memory dumps, as alluded to below.

Resources Required

My estimate of the CUS workspace needed is as follows:

(assuming source files of size about 10kb, and object files of size about 12kb, and executables of size about 300kb)

|  | kb |
| --- | --- |
| 100 source modules and associated interfaces x 3 | 3000 |
| 100 object files | 1200 |
| a few (3) executables | 900 |
| documentation | 500 |
| total | 5600 |

The multiplier of 3 is to allow for the use of the RCS version control system to track changes in the source code.

Hence, as I also expect to use CUS for course work programming examples later in the year, and as my usage already stands at about 2.2Mb, I propose that I need an additional quota of 6Mb on top of my current quota of 5Mb, making a total of 11Mb.

Starting Point

I have no knowledge of the PDP-8/e prior to this year.

There are a few existing emulators written in C available on the World Wide Web. These will be used to evaluate existing test programs which are in BIN loader form and have been obtained from the United States. However, the intention of the project is to start from scratch in Modula-3.

I have available (from the World Wide Web) a cross-assembler and a copy of the BIN loader which runs on a PDP-8/e (emulator) and accepts the assembler's output and loads it into the core of the PDP-8/e. These will be used to run test programs as necessary.

Work Plan

5th to 16th December (2 weeks, no lectures)

Planning and design of the project including decisions about the execution threads required and about the data structures and methods to be used.

Also investigate:

- the feasibility of using FormsVBT to implement the interactive front panel display

- possible methods of implementing the instruction table

- the feasibility and desirability of implementation of various extra features

Implementation of the basic CPU cycle and a minimal instruction set. Include in the program conditionally available diagnostics messages so that the behaviour of the instruction cycle can be traced and so that tests can yield concrete results.

17th December to 8th January: Christmas

9th to 20th January (2 weeks, no lectures)

Implement a simple control interface to allow the running of the emulator from the keyboard. This will require commands to read and write specified memory locations, the ability to start and stop execution, and the ability to load and save core memory from/to disc.

The first test of this interface should be using a harness which simply reports on the commands given to it by the interface. This harness should be kept for use with the graphical interface.

For further testing at this stage the following abilities are required:

- dump sessions to file

- be able to generate a dump of a saved core memory file

- be able to load programs directly into the emulator, via the ability to load core

memory

- diagnostics output from the program as it runs

21st January to 3rd February (2 weeks, with lectures)

Implement high speed tape reader and punch and rudimentary teletype facility, and the CPU instructions required to support them. Implement diagnostics within the CPU instructions to allow easy testing. Test and print test results.

Both should be implemented with files as the source/sink for information. This is not the final intended implementation for the teletype but will allow instruction tests to be performed which output to teletype.

4th February to 3rd March (4 weeks, with lectures)

Implement the rest of the core PDP-8/e instruction set (i.e. that considered necessary for a minimum implementation) plus extra instructions as time permits. Test the implementation. By this stage it should be possible to test using third party test programs.

NB Progress report due at the end of this period (3rd March)

4th to 31st March (4 weeks, two weeks with lectures)

Implement graphical front end:

- design the appearance of the interface

- provide functionality by associating procedures with the items on the graphical front panel

- provide a way to 'plug in' this functionality to the existing front panel

Initial testing of this stage is by use of the test harness used for the simple control interface. Further testing should be by hooking the front end to the rest of the system and checking that key presses perform as expected.

1st to 23rd April: Easter

24th April to 5th May (2 weeks, with lectures)

The above continued.

6th to 12th May (1 week, with lectures)

Implement teletype input from keyboard and output to screen. This is dependent upon the success of the graphical front end - which then frees up the main window for use as the teletype output. Implement the few extra commands required. Test and document test results.

15th May to 18th June: break for revision, exams and celebrations

19th to 30th June (2 weeks, no lectures)

Final testing of program and implementation of other features as time permits.

1st to 30th July (1 month, no lectures)

Write dissertation, print and submit.

The word size was 12 bits.

Jones: PDP-8 Frequently Asked Questions: What is a PDP-8?

Introduction to Programming, page 7-54

See appendix 6 for details of RIM and BIN tape formats.

See appendix 1 for details.

See appendix 2 for full details of instruction formats and memory addressing.

See section 4.1

See sections 3.6 and 4.1.4.

From sources found via Jones' DEC PDP-8 Index.

A picture of the graphical user interface for my emulator can be found between pages 12 and 13.

See appendix 1.

PDP-8/e Small Computer Handbook 1971

PDP-8 Handbook Series: Volume 1: Introduction to Programming

See section 4.2.5.

This holds the opcode of the current instruction.

All transfers to and from memory pass through this register.

This is an internal record of the current values of the switch register switches on the front panel.

This is a one bit register which is set by the SW key on the front panel. On the original machine this key was connected directly to a line on the system bus.

As described in appendix 2.

The FormsVBT Reference Manual, page 60.

See appendix 3.

See appendix 6.

See appendix 3 for details of the initialisation file.

See section 4.2.2.

See section 4.2.3.

This feature is mentioned in section 2.5. The maximum number of instructions is an option which may be specified in the initialisation file: see appendix 3.

See appendix 2 for details of memory addressing.

Hence JMS can read the return address directly from the program counter.

Either by the user or because this was a HLT instruction.

Defined in appendix 5.

See section 2.4.

As described in appendix 5.

See figure 2.

See appendix 3.

See section 3.3.

See section 2.5.

See appendix 2 for details of instruction formats.

As described in chapter 1.

For exact details of individual OPR instructions see the Small Computer Handbook, pages 3-5ff and 4-25ff.

Defined in the Small Computer Handbook, pages 4-27ff.

See section 2.3.

See appendix 2 for details.

This method is separate from the first to simplify the test to decide whether to take an interrupt which is made in the processor cycle.

See section 4.2.

This would be doubled by using a counter of type INTEGER and starting at -232. This would complicate the specification in the initialisation file of the maximum number of instructions to execute.

See appendix 4.

Interrupts ON

Small Computer Handbook, page 3-14.

Having discovered what the correct implementation was by referring to Jones' emulator and the news group alt.sys.pdp8.

See appendix 4.

The teletype's maximum speed was ten characters per second (cps); the paper tape reader read at a maximum of 300 cps. Therefore, with an average instruction time of 2.6 ms, more than 1000 instructions could be executed between characters when reading from paper tape.

The length of time can be varied between runs by specifying it in the initialisation file.

See section 4.2.

This is the token recognition function produced by lex.

As described in section 2.5.

See appendix 5.

See appendix 2.

See appendix 4.

See section 4.1.5.

See section 4.1.6.

See section 3.5.2.

According to Jones' emulator and information obtained via the news group alt.sys.pdp8.

See Small Computer Handbook, page 3-6.

The instructions which are not executed if the implementation is correct are marked *.

See section 3.6 for a description of this problem and its solution.

Here it just counted to a very large number.

See appendix 1 for a description of DEC's tests.

See section 3.6 where the interrupt object methods are described.

As described in section 3.1.

See section 3.1.

The reason for providing this facility is given in section 2.5.

As noted in section 4.2.6.

Small Computer Handbook, pages 4-27ff.

There is a bug in the implementation here as the memory buffer register is usually ignored for reasons of speed so its contents are seldom updated.