UNIX Networking at Purdue

_#I_#n_#t_#r_#o_#d_#u_#c_#t_#i_#o_#n

This paper describes a modest UNIX  network  in  use  and  developed  at  Purdue

University School of Electrical Engineering.  Features include:

o    Virtual terminal access.  Through the program "con" a user can connect  his

physical  terminal  to a "pseudo-terminal" on any other machine.  A virtual

terminal protocol provides local/remote echoing by propagation of stty/gtty

functions.

o    Remote process execution.  The program "csh" (for "connected shell")  takes

as  arguments  a  machine name and a string of one or more commands.  These

commands then execute on the specified machine but with standard input  and

output redirected to the local machine.

o    File transfer/remote device access.  "csh" in  conjunction  with  "cat"  or

"cpio" allows simple file or directory transfers between machines.

o    User programmed net I/O.  The above programs "con" and "csh" access the net

through  a  pool of UNIX "special files".  By issuing "set teletype" (stty)

functions on such an open  file,  programs  can  directly  connect  to  any

machine  in the network, disconnect, wait for connection, send signals, and

so on.  All of these capabilities are available to  any  user-written  pro-

gram.

o    Moderate bandwidth.  Measurements have shown 400K baud process  to  process

within  the  same  machine  and  250K  baud  process to process between two

machines separated by a DMC11 network link.  At this rate single block disk

latency limits any further improvement for file-file transfers.

o    Low system overhead.  Minimal  internal  buffering  and  copying  is  done.

Packets  contain  512 bytes of user data.  The net buffer pool resides out-

side of kernel D space.

o    Generalized machine interconnection.  There are no  master-slave  relation-

ships.   Up to 256 machines can be connected arbitrarily using any combina-

tion of DMC11 and DA11-B network links.  Link device drivers are  reentrant

(code  is  shared  over multiple units).  Packet routing is determined by a

simple static table in each machine at sysgen time.

host

host                          imp

|                             |

|                             |     host

host--imp-----imp--host             imp----imp

|                   host     |

|                            |

(fig A)         host            (fig B)      imp

host

- 2 -

_#M_#a_#c_#h_#i_#n_#e _#i_#n_#t_#e_#r_#c_#o_#n_#n_#e_#c_#t_#i_#o_#n.

In an ideal local network, the intermachine packet switching and  routing  would

take  place  in  a  _#s_#u_#b_#n_#e_#t_#w_#o_#r_#k  of  highly  reliable, single function processors

(interface message processors, "imp"s in ARPA-NET  terminology),  which  connect

through  physically  short  interfaces  with  the "real" machines of the network

(called "host"s in ARPA-NET).  The imps themselves are  then  connected  through

longer  links  to  form  the  backbone or subnetwork (fig A).  Some other subnet

designs have used loop or broadcast techniques for the inter-imp links.

Due to our limited budget and the time it would take to assemble a working  sub-

network,  we have moved the imp function inside of each host, at a cost of added

complexity and reduced network reliability (fig B).

_#S_#p_#e_#c_#i_#a_#l _#f_#i_#l_#e_#s.

Programs such as "con" and "csh", their servers (their counterparts in the other

host which wait for a connection requesting service), and user-written software,

all run in "user" address space as normal processes.  They read and  write  from

network  "special  files"  which  are connected to a special file and process in

another host.

The special files have names /dev/mx/A, /dev/mx/B, /dev/mx/C, and so  on.   "mx"

stands  for multiplex device and the last letter in each filename just selects a

different "minor" device of the "major" mx device.   The  different  file  names

comprise a pool of network files; the actual file name used has no significance.

Before a process can do any network I/O, it must find a free mx file by  opening

them  consecutively  until  a  free  one  is found.  At this point stty's may be

issued to for example connect or wait for a connection; once the  connection  is

open, data written at one end may be read at the other and vice versa.

_#C_#o_#n_#n_#e_#c_#t_#i_#o_#n _#n_#a_#m_#e_#s, _#h_#o_#s_#t _#a_#n_#d _#s_#o_#c_#k_#e_#t _#n_#u_#m_#b_#e_#r_#s.

Hosts are identified by a one byte  host  number.   Within  a  given  host  many

processes  may  wish  to  hold  network conversations simultaneously so a unique

inter-host connection naming convention is needed.  A connection between a  pro-

cess in the local host and a process in a foreign host is uniquely identified by

four numbers, each one byte long: local host, local socket,  foreign  host,  and

foreign  socket.  The socket number can be thought of as specifying a particular

process or service within the given host.

A subset of the socket numbers at each host are reserved for connections to sys-

tem  provided  services.  For example a process called the "con server" (started

initally after the system is booted) is always waiting for connections to  local

socket  number  1.  When a connection arrives the server "fork"s a child process

to handle this particular request while the parent opens a new mx file and  goes

back  to  waiting for further socket 1 connects.  The child now reads and writes

from the connected file (at the other end is the user-initiated  "con"  process)

and  behaves  as  a  pseudo-terminal  for the virtual terminal protocol.  Socket

number 2 is used similarly by the "csh" server.

Note that many connections can exist to the same local host/socket  pair,  since

the  entire  4 byte connection name is still unique.  This is analogous to phone

company PBX service where many incoming calls can be in  progress  to  the  same

number.   Note also that this naming convention eliminates the need for a clumsy

"initial connection protocol".

- 3 -

_#U_#s_#e_#r _#c_#a_#l_#l_#a_#b_#l_#e _#f_#u_#n_#c_#t_#i_#o_#n _#l_#i_#b_#r_#a_#r_#y

A library is available to C programs for performing various  network  functions.

The  library  can  be  loaded with a program by using the "-lmx" switch of the C

compiler (cc) command.  Most of the functions just reformat the  arguments  into

the  appropriate  stty  call on the open network file descriptor supplied as the

first argument.  For completeness the resulting system call which  the  function

makes  is shown in brackets; however users need not be concerned with this level

of detail.  The functions return -1 on failure.

fd = mxfile();  [fd = open("/dev/mx/x",2);]

A free network file from the pool "/dev/mx/x", is found  and  opened.   Its

file  descriptor (fd) is returned.  A valid network file descriptor must be

supplied to most of the functions listed below.  No reads or writes can  be

performed on this fd until a connection is made.

mxwait(fd,socket);  [stty(fd,{3,socket,0});]

Wait for a connection to local socket number "socket" from any host in  the

network.   When  the  connection arrives, mxwait returns.  Socket numbers 1

through 5 are reserved for super-user owned servers.

mxcon(fd,host,socket);  [stty(fd,{1,host,socket});]

Connect the local network file "fd" to foreign host "host", foreign  socket

"socket".   When the connection is complete, mxcon returns.  The connection

is completed if a process at the foreign host has issued the  corresponding

mxwait,  or  issues it at some time in the future.  The local socket number

is picked more or less at random from those unallocated at the local  host.

If  no  path  through  the  network currently exists to "host", mxcon fails

immediately.  The standard UNIX alarm clock signal can be used  to  timeout

mxcon or mxwait, if this is desired.

mxscon(fd,"hostname",socket);

This call is identical to mxcon, except that here a string rather  than  an

integer is used for the host.  A table of standard host names to numbers is

searched and mxcon called with the result.  mxscon fails if the hostname is

not in its table.

mxdis(fd);  [stty(fd,{2,0,0});]

The connected file fd is disconnected; further  reads  or  writes  are  now

ignored.  A new mxcon or mxwait can now be issued on this fd.  A disconnect

occurs automatically on a close(fd) or if the process exits.

read(fd,buffer,count);  write(fd,buffer,count);

Read and write work normally but note the following: (1) Read never returns

more  bytes than were written, and never more than 512 bytes (the host-host

packet size).  In other words the read returns when ANY data is  available;

it  does not wait for the buffer to fill.  (2) So that 1-byte writes (which

are pretty common in UNIX) are not horribly slow, they are buffered locally

inside the kernel until the packet is filled or a timer goes off.

mxsig(fd,signal);  [stty(fd,{6,signal,0});]

- 4 -

The process at the other end of the connection is sent a UNIX signal number

"signal".  See the UNIX programmers manual section SIGNAL(II).

mxpgrp(fd);  [stty(fd,{5,0,0});]

The current process and all its children by "fork"s are placed in the  same

unique  "process group".  If a signal (mxsig) is received over the net, all

the members of the process group receive it.

mxeof(fd);  [stty(fd,{7,0,0});]

An end of file (EOF) is written onto the connected file fd, such  that  all

reads  issued  at  the  other  end will now return a zero byte count (EOF).

Further writes at this end are ignored.

mxserve(socket);

This call is used mainly by the con and csh servers to answer requests  for

service on socket number "socket".  The basic actions are:

close all file descriptors

while(1) {

mxfile();               /* fd will = 0 */

mxwait(0,socket);

fork();                 /* split into 2 processes */

if(child process) {

dup(0);  dup(0);     /* fd 0,1,2 now net file */

return;

}

if(parent process) {

close(0);

}

}

Since most UNIX processes do input from file descriptor 0 (standard  input)  and

output  to file descriptor 1 (standard output), such a process if "exec"ed after

calling mxserve will do its I/O over the network.

---------------------------

|network application progs|

|con, csh servers         |      user space

|=========================|      =======

|mx device driver         |      kernel space

|/dev/mx/x special files  |

|- - - - - - - - - - - - -|

|net interface control    |

|(host-host protocol)     |

|-------------------------|

|      |------------------|

| imp  | line driver da   |--->

|      |------------------|

|      | line driver dmc  |--->  to other machines

|      |------------------|

(fig C)   |      | line driver dmc  |--->

---------------------------

- 5 -

_#O_#v_#e_#r_#v_#i_#e_#w _#o_#f _#i_#n_#t_#e_#r_#n_#a_#l _#o_#r_#g_#a_#n_#i_#z_#a_#t_#i_#o_#n

The network software in each machine is split into two parts (fig C):

IMP  The "imp" process receives buffers (packets) from the local host  and  from

neighboring  imps.  The imp examines the destination address on each packet

it receives and looks up the host number in  its  (static)  routing  table.

This  table  maps host numbers to external line (link) numbers.  The packet

is then enqueued for output via the appropriate line driver and the  driver

is  activated  if idle.  A special routing table entry marks the local host

number.  Packets for the local host are passed directly  via  a  subroutine

call to the mx driver.

MX   The mx device (/dev/mx/x) appears as a conventional device driver to  UNIX.

Open, close, read, write, and stty calls on mx files pass control to the mx

driver in kernel space.  The driver generates packets  containing  host  to

host protocol and passes these to the imp for delivery.  Host to host pack-

ets from foreign hosts are received from the imp and decoded.

_#H_#o_#s_#t _#t_#o _#h_#o_#s_#t _#p_#r_#o_#t_#o_#c_#o_#l.

Hosts communicate with each other by exchanging packets containing host to  host

protocol.   The  structure of a host to host packet is shown below (fig D).  The

first four bytes can be seen as the connection name.  "byte_count" contains  the

number  of  bytes  in  the  array data[], if any.  The maximum number of bytes a

packet can contain is 512, which is the same size as a disk block.

struct packet {

char   source_host;

char   source_socket;

char   dest_host;

char   dest_socket;

char   host_control;              (fig D)

char   imp_control;

int    byte_count;

char   data[512];

}

The imp and host control fields are usually zero, in which case this is a packet

of  "byte  count" bytes of user data from the source host/socket to the destina-

tion host/socket given in the header.

The host_control field when non-zero is a small integer op-code which represents

a  host-host  control  message.   Below are listed the op-code mnemonics and the

function a packet with that op-code performs.

con  connect.  This packet requests that the connection named in the first  four

bytes  be  established.   The  connection  is  open when a pair of these is

exchanged, one in each direction.  After the connection is open, data pack-

ets  may  then  be  sent.  If the host receiving this request has a process

with a matching mxwait(fd,socket) pending, the matching con is sent and the

connection  is  open.  If no one is currently listening (mxwait pending) to

that socket at the receiver, the con is remembered but no  reply  is  sent.

An  mxwait  then  issued  some time later will find this remembered con and

open the connection by replying.

- 6 -

Once a con has been sent, that connection name must not be used again until

disconnects  (dis,  below) are exchanged.  If a con is sent and no matching

con returns, the user or process which issued the mxcon (causing con to  be

sent)  will  typically  time  out and close or mxdis the offending mx file,

which causes dis to be sent.

[Implementation note: Rather than using a giant ARPA-NET-style finite-state

machine  to record the state of connection establishment, it was found suf-

ficient to just store a bit vector: (con sent, con received, dis sent,  dis

received)  in each host per connection, and just make tests on combinations

of these bits.]

dis  disconnect.  This requests that the connection  named  in  the  first  four

bytes  be  broken.   The  disconnect  is  complete when a pair of these are

exchanged.

next ready for next packet.  After a data packet is sent and  before  more  data

can  be sent, this op-code must be received.  The "next" op-code is sent by

the consumer of the data packet when the user process at that end reads the

data  from  kernel  into  user  space.   This flow-control scheme is crude,

half-duplex, and is the reason we can only get 250k baud out of our 1m baud

DMC11;  but  it's  simple  and reasonably effective due to the large packet

size used.  [Of course the remaining 750k  bandwidth  is  available  for  3

other connections doing simultaneous full-speed transfers.]

sig  signal.  Sends interrupt signal number in data[0], to the  process  at  the

other  end  of  the  connection.   Note that this bypasses the queued data-

packet stream.

rst  reset.  The source and dest socket numbers are not used.  The  source  host

is  telling  the  dest  host  to  clear out and reset all known connections

between the two.  This is "broadcast" by each host when it comes  up  (nor-

mally or after a crash) to clean up loose ends.

rrp  reset reply.  Reply to rst op-code.  Not currently implemented.

_#I_#m_#p-_#h_#o_#s_#t _#p_#r_#o_#t_#o_#c_#o_#l.

Rather than adding another independent layer of protocol on top of host-host,  a

single  byte is stolen from the host-host packet header, and when non-zero indi-

cates an imp to host or host to imp control op-code.  There  is  currently  only

one op-code and it is imp to host:

dead dead host.  This packet is being returned because there is no path  to  the

host it was sent to.  The imp that discovered this fact simply reversed the

source/dest fields, set this op-code in the header, and requeued the packet

for  transmission.   The second time a packet bounces off an imp like this,

it is destroyed, for both the source and dest host are then dead.

_#I_#m_#p-_#i_#m_#p _#p_#r_#o_#t_#o_#c_#o_#l.

There is no single imp-imp protocol at present but rather a  different  protocol

used by the line device drivers for each type of physical link.  There is a more

or less common type of imp-imp header that is prefixed to the  host-host  packet

header.   It  contains  a  start  of  header (SOH) or flag character, a sequence

number, and an optional checksum.  Currently DMC11 and DA11-B line  drivers  are

supported, a DH11 driver will be available in the near future.

- 7 -

_#I_#m_#p _#p_#r_#o_#c_#e_#s_#s / _#l_#i_#n_#e _#d_#r_#i_#v_#e_#r _#i_#n_#t_#e_#r_#a_#c_#t_#i_#o_#n.

The imp process is activated whenever a packet arrives at the imp,  from  either

the  local  host  or from neighboring imps.  The packets are placed on the imp's

single "task" (or work) queue and the imp activated by a level 1 program  inter-

rupt  request.  The imp: (1) dequeues packets one at a time;  (2) examines their

destination;  (3) checks that the appropriate line is "alive"; (4)  places  them

on that line driver work queue; and (5) activates the driver.

Each line driver has a standard set of entry points:

TPACK    A packet has been placed on the drivers work queue.

TTIMER   Each driver is entered here 10 times per second for use in timeouts and

in determining if the physical link is "alive".

TSTART   Entered at network start-up time  to  handle  preset  details:  initial

buffer allocation, csr setup, etc.

(inter)  Each driver has one or two interrupt entry points from the vectors  (in

low.s).   Packets  received for the imp at this level are passed to the

imp through his task queue.

The "unit" number is an argument in all of  these  entry  points  to  allow  the

driver code to be shared over multiple physical link devices of the same type.

_#D_#M_#C_#1_#1 _#l_#i_#n_#e _#d_#r_#i_#v_#e_#r.

The DMC microcomputer does most of the work but some code is still required for:

o    sequence number and general consistency checking.  It might be due  to  the

old  ROM set in our DMC, but several DMC bugs were found: On rare occasions

the DMC will loose a buffer with no error status.  Two  or  three  times  a

weird  trap  has occured on one cpu when the cpu at the other end is halted

(perhaps in the middle of a DMC dma).   Defective  devices  on  the  unibus

which  hog the bus for too long cause problems.  The sequence and timing of

commands to first-time initialize  the  DMC  were  found  to  be  critical.

Internal  UNIX  errors  which lockout interrupts during /dev/tty8 (console)

error messages make the DMC sick.  (We have improved error message handling

to avoid most of these lockouts).

o    idle packets.  The old ROM set also requires something to  be  sent  fairly

often  to keep the line responsive.  DECs new ROMs will eliminate this (and

we hope the other ideosyncracies).  Idle packets also are  used  to  detect

the line coming alive.

Although the DMC has its little quirks, now that we have figured them out we are

very pleased with its performance and reliability.

_#D_#A_#1_#1-_#B _#l_#i_#n_#e _#d_#r_#i_#v_#e_#r.

The imp-imp links are logically full-duplex channels with no master-slave  rela-

tionship.  Since the DA is half-duplex, code had to be provided to give the phy-

sical link this logical appearance.  Both sides must cooperate at the  beginning

and end of each packet transfer.

Each side is always in one of 4 states:  (1) interrupts off (for about a  second

- 8 -

after  an  INIT pulse is detected, to prevent continuous interrupts).  (2) idle.

(3) receive.  (4) transmit.  Each side remains in idle state until  either:  (1)

the local side gets a packet to send from its imp, in which case the local state

goes from idle to transmit while a dma transmit is started and a  "transmit  go"

interrupt  sent  to  the  other side.  (2) a "transmit go" interrupt is received

from the other side causing this side to go from idle to  receive  and  start  a

receive dma.

Along with the "transmit go" signal the word count  is  sent;  this  allows  the

receiver  to  setup  his dma word count properly.  The transmitter actually sets

his dma count one word higher (in fact 2 words higher for  obscure  reasons)  so

that  the receive side always services his interrupt first upon transfer comple-

tion.  Upon completion the receiver decides if he now wants to transmit a packet

(perhaps queued during the receive transfer).  If so the receiver moves directly

to transmit state (sending a "transmit go" to the previous transmitter), if  not

receive   moves  to  idle,  sending  a  completion  interrupt  to  the  previous

transmitter.  This scheme allows packets to flow alternately in  each  direction

if transmit queues exist on both sides.

Of course occasionally both sides will decide to  transmit  at  the  same  time.

Since  the  hardware  does  not contain any arbitration logic for this case, the

software must detect this state and correct it.  At  sysgen  time  one  side  is

declared  "secondary" and the other "primary".  In the event of a collision, the

hardware hang is cleared and the primary allowed to go first.

Sequence numbers are used to detect missing blocks (from gross hardware faults),

but  no  checksum  is used since the physical distance involved is less than 100

feet.  Timeouts are used to send null blocks every second  for  line  alive/dead

determination.   Collisions  are  detected and corrected immediately, but rarely

the link freezes unexplicably (a cryptic comment concerning this was  discovered

in a DEC written DA11 driver), so another timeout trys to thaw things out.

Although the DA11-B is now being used, we think half-duplex links are just  gen-

erally  undesirable  and would recommend the DMC over the DA any day.  Only when

one is stuck with an old DA (as we were) should this link driver be used.

_#V_#i_#r_#t_#u_#a_#l _#t_#e_#r_#m_#i_#n_#a_#l _#p_#r_#o_#t_#o_#c_#o_#l ("_#c_#o_#n" _#p_#r_#o_#g_#r_#a_#m)

When "con" is in use there are actually 4  processes  and  a  "pseudo  terminal"

involved:

local host | foreign host

|                        ---------

keyboard  (fd0) ---> con (send) >----|----> s_con (recv) ---> |   |   |

|                        |pty|tty|

screen    (fd1) <--- con (recv) <----|----< s_con (send) <--- |   |   |

|                        ---------

(fig E)                              |

local    The con at the local host is split into two parts: one reads  from  the

keyboard  (fd0)  and  writes to the net, while the other reads from the

net and writes to the screen (fd1).

foreign  At the foreign host the "con server" (called s_con) has also split into

two  parts.   One  half is reading from the net and writing to a pseudo

terminal, the other half is reading from the same pseudo  terminal  and

- 9 -

writing to the net.

A pseudo terminal is a device driver that has two sides.  For  a  single  pseudo

terminal  the sides might be named /dev/ttyx and /dev/ptyx.  Anything written on

/dev/ptyx looks like it was "typed in" at /dev/ttyx, while  everything  "printed

out" at /dev/ttyx can be read at /dev/ptyx.

The con server's job is very simple; when a connect arrives on socket  1,  s_con

forks once to generate a child.  The child s_con then finds a free /dev/ptyx and

forks into receiving and sending parts.

Certain "escape" or "command" character sequences are generated and accepted  by

/dev/ptyx.   For  example, when an "stty" function is issued on /dev/ttyx, it is

translated to a command sequence that is then read from  /dev/ptyx.   The  local

con  program  and  /dev/ptyx  use  this "virtual terminal" protocol to propagate

stty/gtty's and to perform other operations.  The con server (s_con) only issues

reads  and  writes,  it never intercepts the command sequences between /dev/ptyx

and con.

Each "command" consists of an escape byte called "IAC" (interpret  as  command),

followed  by  a  command  code byte, possibly followed by data for that command.

The mnemonics for the commands are listed below with their function.

ST   set teletype, followed  by  6  bytes  from  the  stty  function.   Sent  by

/dev/ptyx  when  an  stty  is  issued  on  /dev/ttyx.   Also sent by con in

response to a GT (below).

GT   get teletype.  Sent by /dev/ptyx when a gtty is issued on  /dev/ttyx.   The

user process that issued the gtty sleeps until ST returns from con.

IN   interrupt signal.  Sent by con when an actual interrupt (not  just  rubout)

is  generated  by  the  user.   A rubout typed by the user in raw mode thus

causes no problems.

QU   quit signal.

EF   end of file.

DM   data mark.  When con detects local interrupt or quit, and the IAC IN or  QU

sequence is sent, a flag is set so that all data characters received by con

from the net are flushed   --until  this  command  (IAC  DM)  is  received.

/dev/ptyx  sends  the data mark when IN or QU is received, to flush out the

output stream to the other host.

IAC  an IAC sent as data must be doubled.

_#R_#e_#m_#o_#t_#e _#p_#r_#o_#c_#e_#s_#s _#p_#r_#o_#t_#o_#c_#o_#l ("_#c_#s_#h" _#p_#r_#o_#g_#r_#a_#m)

The sequence of events when using csh is as follows:

1.   The local csh connects to socket 2 on the foreign host.  It then  writes  3

lines  (each '\n' terminated but as a single write, for speed): name, pass-

word, and command line.  Name and password are possibly null.

2.   At the foreign host, s_csh (csh server) has received the socket  2  connec-

tion and forks a child to handle it.

- 10 -

3.   The child s_csh has file descriptors 0,1, and 2 (standard  I/O  and  error)

open  as  the  net  file.  s_csh reads the name, password, and command.  If

name/password is not null, s_csh "logs in" as that  user  name,  else  user

name "user" is the default.

4.   s_csh now "exec"s in the command on top of itself.  The command will do its

I/O from the net.

5.   Meanwhile the local csh has split into two parts, one reads  from  standard

input  and writes to the net, while the other reads from the net and writes

on standard output.  When the half that is reading standard input  gets  an

EOF, it writes an EOF to the net and exits.

6.   The command running at the foreign host will eventually exit.

7.   The local csh reading the net will get an EOF when the command exits at the

foreign host;  the local csh then exits.

_#K_#e_#r_#n_#e_#l _#n_#e_#t_#w_#o_#r_#k _#b_#u_#f_#f_#e_#r_#s.

Memory management register 5 (ka5) in our UNIX kernel has been freed for use  as

a  general  window  into  extended  disk, network, and misc. buffer areas.  This

allows a healthy supply of dedicated network buffers.

The net buffers have no "buf" structure associated with them (as do system  disk

buffers)  and  are large enough to hold a packet complete with internal, imp-imp

and host-host headers.  This means user  data  can  be  moved  directly  to  its

transmission buffer in kernel space from user space ONCE, with no further moves.

The buffers are located on 64 byte (memory management) boundaries so  that  they

are  easily  referenced.   Buffers  in  queues are circularly linked (queues are

named by their tails), and links (which are stored in the  first  word)  can  be

plugged  directly into *ka5 to access that buffer (links are the physical memory

address shifted right 6 bits).

_#C_#u_#r_#r_#e_#n_#t _#c_#o_#n_#f_#i_#g_#u_#r_#a_#t_#i_#o_#n

Our site currently has 2 11/70s and an 11/45 connected; the /70s by a DMC11, and

the  /45  to one /70 with a DA11-B.  Three other /45s in the same building could

be connected immediately if we had 3 pairs of DMC11's.  Currently we are working

on a low speed serial (19k baud) link driver for these machines.

_#A_#v_#a_#i_#l_#a_#b_#i_#l_#i_#t_#y

The mx software is public domain and free  to  anyone  with  a  version  6  UNIX

license.   Currently  we  only have ours running on split I/D machines:  11/45s,

11/70s, etc.  It is not known whether it could be squeezed into an 11/40 or /60.

Minor  mods  would also be needed in machines without the programmable interrupt

request hardware to perform this with software (we  were  just  lazy).   Perhaps

more  critical  are  the  modifications we have made to the "standard" version 6

kernel and user space programs.

o    We no longer have "groups".  The group byte in the  inode  has  been  taken

over  by  the high order bits of the 16 bit user id (we have a large number

of users).

- 11 -

o    The ka5 window is used for:  most system disk  buffers,  net  buffers,  the

proc table, the clist, and high speed (low sys time) terminal output.

o    The "speeds" word of the stty/gtty function has been rearranged to  support

various  local  serial  I/O needs:  Full speed 19K baud input to a circular

buffer in user space.  APL overstrikes on specially modified ADM3As.  Etc.

o    Powerfail recovery and retry.  Device drivers are entered with dev == NODEV

following a powerfail.  99% of our outages are now automatically recovered.

o    A lot of "little" things that improve throughput and system error recovery.

Most  of  the  "PWB  diff  listing"  kernel  mods  (obtained  from UUG) are

installed.

To run the mx code we really recommend running with our kernel as well.  Many of

the  mods  above  can  be  conditionally  compiled out, but some cannot (such as

uid's, we didnt know of #ifdef back then).  Thus installing only "part"  of  our

kernel will require some effort.