To:	Users

From:	The Author

Subj:	Simulator Usage, V2.2d

Date:	1-Dec-96

This memorandum documents the PDP-8, PDP-11, PDP 18b, Nova, and IBM 1401

simulators.  These simulators are freeware; commercial use is prohibited,

and support is not available.  The best way to fix problems or add features

is to read and modify the sources yourself.  Alternately, you can send

Internet mail to bob.supnik@ljo.dec.com, but responses are not guaranteed.

The simulators use a common command interface.  The memorandum first

describes the common features of the command interface and then provides

information on each of the individual simulators.

1. Compiling And Running A Simulator

The simulators have been tested on VAX VMS, Alpha VMS, Alpha UNIX, Intel

and LINUX; a port to Windows 95, and Windows NT is in progress but lacks

terminal emulation code.  Porting to other environments will require

changes to the operating system dependent code in scp_tty.c.

To compile the simulators on VMS, use these commands (note that separate

compilations are required for each of the 18b PDP's):

$ cc pdp8_*.c,scp.c,scp_tty.c		! PDP-8

$ link/exec=pdp8 pdp8_*.obj,scp.obj,scp_tty.obj

$ cc pdp11_*.c,scp.c,scp_tty.c		! PDP-11

$ link/exec=pdp11 pdp11_*.obj,scp.obj,scp_tty.obj

$ cc nova_*.c,scp.c,scp_tty.c		! Nova

$ link/exec=nova nova_*.obj,scp.obj,scp_tty.obj

$ cc/define=PDP{4,7,9,15} pdp18b_*.c,scp.c,scp_tty.c

$ link/exec=pdp{4,7,9,15} pdp18b_*.obj,scp.obj,scp_tty.obj

$ cc i1401_*.c,scp.c,scp_tty.c		! IBM 1401

$ link/exec=i1401 i1401_*.obj,scp.obj,scp_tty.obj

On version of VMS prior to 6.2, the simulators must then be defined as

foreign commands so that they can be started by name.

To compile the simulators on Alpha UNIX, use these commands (note that

separate compilations are required for each of the 18b PDP's):

% cc pdp8*.c scp*.c -lm -o pdp8

% cc pdp11*.c scp*.c -o pdp11

% cc nova*.c scp*.c -lm -o nova

% cc -DPDP{4,7,9,15} pdp18b*.c scp*.c -lm -o pdp{4,7,9,15}

% cc i1401*.c scp*.c -o i1401

To compile the simulators on LINUX, use these commands:

% cc -DLINUX pdp8*.c scp*.c -lm -lbsd -o pdp8

% cc -DLINUX pdp11*.c scp*.c -lbsd -o pdp11

% cc -DLINUX nova*.c scp*.c -lm -lbsd -o nova

% cc -DPDP{4,7,9,15} -DLINUX pdp18b*.c scp*.c -lm -o pdp{4,7,9,15}

% cc -DLINUX i1401*.c scp*.c -o i1401

To start the simulator, simply type its name.  The simulator takes

one optional argument, a startup command file.  If specified, this

file should contain a series of non-interactive simulator commands,

one per line.  These command can be used to set up standard parameters,

for example, disk sizes.

% pdp8 <startup file>(cr)		or

% pdp11 <startup file>(cr)		or

% nova <startup file>(cr)		or

% pdp{4,7,9,15} <startup file>(cr)	or

% i1401 <startup file>(cr)

The simulator types out its name and version, executes the commands

in the startup file, if any, and then prompts for input with

sim>

2. Simulator Conventions

A simulator consists of a series of devices, the first of which is always

the CPU.  A device consists of named registers and one or more numbered

units.  Registers correspond to device state, units to device address

spaces.  Thus, the CPU device might have registers like PC, ION, etc,

and a unit corresponding to main memory; a disk device might have

registers like BUSY, DONE, etc, and units corresponding to individual

disk drives.  Except for main memory, device address spaces are simulated

as unstructured binary disk files in the host file system.  (Thus, files

with data widths greater than 8b are endian dependent.)  The SHOW CONFIG

command displays the simulator configuration.

A simulator keeps time in terms of arbitrary units, usually one time unit

per instruction executed.  Simulated events (such as completion of I/O)

are scheduled at some number of time units in the future.  The simulator

executes synchronously, invoking event processors when simulated events

are scheduled to occur.  Even asynchronous events, like keyboard input,

are handled by polling at synchronous intervals.  The SHOW QUEUE command

displays the simulator event queue.

3. Commands

3.1 Loading Programs

The LOAD command (abbreviation LO) loads a file in binary paper-tape

loader format:

sim> load <filename>(cr)

3.2 Saving and Restoring State

The SAVE command (abbreviation SA) save the complete state of the

simulator to a file.  This includes the contents of main memory and

all registers, and the I/O connections of devices:

sim> save <filename>(cr)

The RESTORE command (abbreviation REST, alternately GET) restores a

previously saved simulator state:

sim> restore <filename>(cr)

Note: SAVE file format compresses zeroes to minimize file size.

3.3 Resetting Devices

The RESET command (abbreviation RE) resets a device or the entire

simulator to a predefined condition:

sim> RESET(cr)			-- reset all devices

sim> RESET ALL(cr)		-- reset all devices

sim> RESET <device>(cr)		-- reset specified device

Typically, RESET stops any in-progress I/O operation, clears any

interrupt request, and returns the device to a quiescent state.  It

does not clear main memory or affect I/O connections.

3.4 Connecting and Disconnecting Devices

Except for main memory, simulated unit address spaces are simulated as

unstructured binary disk files in the host file system.  Before using a

simulated unit the user must specify the file to be accessed by that

unit.  The ATTACH (abbreviation AT) command associates a unit and a file:

sim> ATTACH <device><unit number> <filename>(cr)

If the file does not exist, it is created, and an appropriate message

is printed.

The DETACH (abbreviation DET) command breaks the association between a

unit and a file and closes the file:

sim> DETACH ALL(cr)			-- detach all units

sim> DETACH <device><unit number>(cr)	-- detach specified unit

The EXIT command performs an automatic DETACH ALL.

3.5 Examining and Changing State

There are four commands to examine and change state:

EXAMINE (abbreviated E) examines state

DEPOSIT (abbreviated D) changes state

IEXAMINE (interactive examine, abbreviated IE) examines state

and allows the user to interactively change it

IDEPOSIT (interactive deposit, abbreviated ID) allows the user

to interactively change state

All four commands take the form

command {-switches} {<device>{<unit number>}} <state list>

Deposit must also include a deposit value at the end of the command.

The examine and deposit commands can work on the address space of any

unit.  If no device is specified, the CPU (main memory) is selected; if

a device but no unit is specified, unit 0 of the specified device is

selected.  The "state list" consists of one or more of the following,

separated by commas:

register		the specified register

register1-register2	all the registers starting at register1

up to and including register2

address			the specified location

address1-address2	all locations starting at address1 up to

and including address2

STATE			all registers in the device

ALL			all locations in the unit

Switches can be used to control the format of display information:

-a			display as ASCII

-c			display as character string

-m			display as instruction mnemonics

The simulators typically accept symbolic input (see simulator sections).

Examples:

sim> ex 1000-1100		-- examine 1100:1100

sim> de PC 1040			-- set PC to 1040

sim> ie 40-50			-- interactively examine 40:50

sim> ex rx0 50060		-- examine 50060, RX unit 0

sim> de all 0			-- set main memory to 0

Note: to terminate an interactive command, simply type a bad value (eg,

XXX) when input is requested.

3.6 Running Programs

The RUN command (abbreviated RU) resets all devices, deposits its argument

(if given) in the PC, and starts execution.  If no argument is given,

execution starts at the current PC.

The GO command does NOT reset devices, deposits its argument (if given) in

the PC, and starts execution.  If no argument is given, execution starts at

the current PC.

The CONT command (abbreviated CO) does NOT reset devices and resumes

execution at the current PC.

The STEP command (abbreviated S) resumes execution at the current PC for

the number of instructions given by its argument.  If no argument is

supplied, one instruction is executed.

The BOOT command (abbreviated BO) bootstraps the device and unit given

by its argument.  If no unit is supplied, unit 0 is boostrapped.  The

specified unit must be attached to a file.

3.7 Stopping Programs

Programs run until the simulator detects an error or stop condition, or

until the user forces a stop condition.

3.7.1 Simulator Detected Stop Conditions

These simulator-detected conditions stop simulation:

o HALT instruction.  If a HALT instruction is decoded, simulation stops.

o Breakpoint.  The IBKPT register provides a single virtual address

breakpoint.  If the PC matches the contents of the IBKPT register,

simulation stops.  The breakpoint is automatically disabled for

the next instruction execution.

o I/O error.  If an I/O error occurs during simulation of an I/O operation,

and the device stop-on-I/O-error flag is set, simulation usually stops.

o Processor condition.  Certain processor conditions can stop simulation;

these are described with the individual simulators.

3.7.2 User Specified Stop Conditions

Typing the interrupt character stops simulation.  The interrupt character

is defined by the WRU (where are you) register and is initially set to

005 (^E).

3.8 Setting Device Parameters

The SET command (abbreviated SE) changes the status of a device parameter:

sim> SET <unit> <parameter>

The parameters are simulator and device specific.  Disk drives, for

example, can usually be set write ENABLED or write LOCKED; if a device

supports multiple drive types, the SET command can be used to specify

the drive type.

3.9 Displaying Parameters and Status

The SHOW CONFIGURATION command shows the simulator configuration and the

status of all simulated I/O devices.

The SHOW <device> command shows the status of the named simulated I/O

device.

The SHOW QUEUE command shows the state of the simulator event queue.  Times

are in "simulation units", typically one unit per instruction execution,

relative to the current simulation time.

The SHOW TIME command shows the number of time units elapsed since

the last RUN command.

3.10 Altering the Simulated Configuration

In devices with multiple units, the REMOVE <unit> command removes the

specified unit from the configuration.  Once removed, a unit cannot be

manipulated in any way until it is added back to the configuration.

ADD <unit> adds back a unit that had been removed from the configuration.

3.11 Exiting The Simulator

EXIT (synonyms QUIT and BYE) returns control to the operating system.

4. PDP-8 Features

The PDP-8 simulator is configured as follows:

device		simulates

name(s)

CPU		PDP-8/E CPU with 32KW of memory

-		KE8E extended arithmetic element (EAE)

-		KM8E memory management and timeshare control

PTR,PTP		PC8E paper tape reader/punch

TTI,TTO		KL8E console terminal

LPT		LE8E line printer

CLK		DK8E line frequency clock (also PDP-8/A compatible)

RK		RK8E/RK05 cartridge disk controller with four drives

RF		RF08/RS08 fixed head disk controller with four platters

RX		RX8E/RX01 floppy disk controller with two drives

MT		TM8E/TU10 magnetic tape controller with eight drives

The PDP-8 simulator implements one unique stop condition: if an undefined

instruction (unimplemented IOT or OPR) is decoded, and register STOP_INST

is set, the simulator halts.

4.1 CPU

The only CPU options are the presense of the EAE and the size of main

memory; the memory extension and time-share control is always included,

even if memory size is 4K.

SET CPU EAE		enable EAE

SET CPU NOEAE		disable EAE

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 32K.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		15	program counter, including IF as high 3 bits

AC		12	accumulator

MQ		12	multiplier-quotient

L		1	link

SR		12	front panel switches

IF		3	instruction field

DF		3	data field

IB		3	instruction field buffer

SF		7	save field

UF		1	user mode flag

UB		1	user mode buffer

SC		5	EAE shift counter

GTF		1	EAE greater than flag

EMODE		1	EAE mode (0 = A, 1 = B)

ION		1	interrupt enable

ION_DELAY	1	interrupt enable delay for ION

CIF_DELAY	1	interrupt enable delay for CIF

PWR_INT		1	power fail interrupt

UF_INT		1	user mode violation interrupt

INT		15	interrupt pending flags

DONE		15	device done flags

ENABLE		15	device interrupt enable flags

OLDPC		15	PC prior to last JMP, JMS, or interrupt

STOP_INST	1	stop on undefined instruction

BREAK		16	breakpoint address (177777 to disable)

WRU		8	interrupt character

4.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The teletype reads and writes to the controlling console port.  The

keyboard has one option, UC; when set, it automatically converts lower

case input to upper case.  This is required by OS/8.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

INT		1	interrupt pending flag

DONE		1	device done flag

ENABLE		1	interrupt enable flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		31	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

4.3 RK8E/RK05

RK8E options include the ability to make units write enabled or write locked:

SET RKn LOCKED		set unit n write locked

SET RKn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The RK8E implements these registers:

name		size	comments

STA		12	status

DA		12	disk address

MA		12	current memory address

CMD		12	disk command

BUSY		1	control busy flag

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

4.4 RX8E/RX01

RX8E options include the ability to set units write enabled or write locked:

SET RXn LOCKED		set unit n write locked

SET RXn ENABLED		set unit n write enabled

The RX8E implements these registers:

name		size	comments

RXCS		12	status

RXDB		12	data buffer

RXES		8	error status

RXTA		8	current track

RXSA		8	current sector

STAPTR		3	controller state

BUFPTR		3	buffer pointer

INT		1	interrupt pending flag

DONE		1	device done flag

ENABLE		1	interrupt enable flag

TR		1	transfer ready flag

ERR		1	error flag

CTIME		24	command completion time

STIME		24	seek time, per track

XTIME		24	transfer ready delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

4.5 RF08/RS08

The RF08 implements these registers:

name		size	comments

STA		12	status

DA		20	current disk address

MA		12	memory address (in memory)

WC		12	word count (in memory)

WLK		32	write lock switches

INT		1	interrupt pending flag

DONE		1	device done flag

TIME		24	rotational delay, per word

BURST		1	burst flag

STOP_IOE	1	stop on I/O error

The RF08 is a three-cycle data break device.  If BURST = 0, word transfers

are scheduled individually; if BURST = 1, the entire transfer occurs in

a single data break.

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RF08 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

4.6 Magnetic Tape (MT)

Magnetic tape options include the ability to make units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The magnetic tape controller implements these registers:

name		size	comments

CMD		12	command

FNC		12	function

CA		12	memory address

WC		12	word count

DB		12	data buffer

STA		12	main status

STA2		6	secondary status

DONE		1	device done flag

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

TIME		24	record delay

UST0..7		24	unit status, units 0..n

POS0..7		31	position, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

4.7 Symbolic Display and Input

The PDP-8 simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as (sixbit) character string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command

line switches:

' or -a			ASCII character

" or -c			two character sixbit string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard PDP-8 assembler syntax.  There are four

instruction classes: memory reference, IOT, field change, and operate.

Memory reference instructions have the format

memref {I} {T/Z} address

where I signifies indirect, T a current page reference, and Z a zero page

reference.  The address is an octal number in the range 0 - 07777; if T or

Z is specified, the address is a page offset in the range 0 - 177.  Normally,

T is not needed; the simulator figures out from the address what mode to use.

However, when referencing memory outside the CPU (eg, disks), there is no

valid PC, and T must be used to specify current page addressing.

IOT instructions consist of single mnemonics, eg, KRB, TLS.  IOT instructions

may be or'd together

iot iot iot...

The simulator does not check the legality of the proposed combination.  IOT's

for which there is no opcode may be specified as IOT n, where n is an octal

number in the range 0 - 0777.

Field change instructions (CIF, CDF) have the format

fldchg field

where field is an octal number in the range 0 - 7.  Field change instructions

may be or'd together.

Operate instructions have the format

opr opr opr...

The simulator does not check the legality of the proposed combination.  EAE

mode A and B mnemonics may be specified regardless of the EAE mode.  The

operands for MUY and DVI must be deposited explicitly.

5. PDP-11 Features

The PDP-11 simulator is configured as follows:

device		simulates

name(s)

CPU		J-11 CPU with 256KB of memory

-		FP11 floating point unit (FPA)

PTR,PTP		PC11 paper tape reader/punch

TTI,TTO		DL11 console terminal

LPT		LP11 line printer

CLK		line frequency clock

RK		RK11/RK05 cartridge disk controller with eight drives

RL		RLV12/RL01(2) cartridge disk controller with four drives

RX		RX11/RX01 floppy disk controller with two drives

TM		TM11/TU10 magnetic tape controller with eight drives

The PDP-11 simulator implements several unique stop conditions:

- abort during exception vector fetch, and register STOP_VEC is set

- abort during exception stack push, and register STOP_SPA is set

- trap condition 'n' occurs, and register STOP_TRAP<n> is set

- wait state entered, and no I/O operations outstanding

(ie, no interrupt can ever occur)

5.1 CPU

The only CPU options are disabling of 22b addressing and the size of main

memory.

SET CPU 18B		disable 22b addressing

SET CPU 22B		enable 22b addressing

SET CPU 16K		set memory size = 16KB

SET CPU 32K		set memory size = 32KB

SET CPU 48K		set memory size = 48KB

SET CPU 64K		set memory size = 64KB

SET CPU 96K		set memory size = 96KB

SET CPU 128K		set memory size = 128KB

SET CPU 192K		set memory size = 192KB

SET CPU 256K		set memory size = 256KB

SET CPU 384K		set memory size = 384KB

SET CPU 512K		set memory size = 512KB

SET CPU 768K		set memory size = 768KB

SET CPU 1024K (or 1M)	set memory size = 1024KB

SET CPU 2048K (or 2M)	set memory size = 2048KB

SET CPU 3072K (or 3M)	set memory size = 3072KB

SET CPU 4096K (or 4M)	set memory size = 4096KB

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 256KB.

These switches are recognized when examining or depositing in CPU memory:

-v			interpret address as virtual

-d			if mem mgt enabled, force data space

-k			if mem mgt enabled, force kernel mode

-s			if mem mgt enabled, force supervisor mode

-u			if mem mgt enabled, force user mode

-p			if mem mgt enabled, force previous mode

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		16	program counter

R0..R5		16	R0..R5, first register set

R10..R15	16	R0..R5, second register set

KSP		16	kernel stack pointer

SSP		16	supervisor stack pointer

USP		16	user stack pointer

PSW		16	processor status word

CM		2	  current mode, PSW<15:14>

PM		2	  previous mode, PSW<13:12>

RS		2	  register set, PSW<11>

IPL		3	  interrupt priority level, PSW<7:5>

T		1	  trace bit, PSW<4>

N		1	  negative flag, PSW<3>

Z		1	  zero flag, PSW<2>

V		1	  overflow flag, PSW<1>

C		1	  carry flag, PSW<0>

SR		16	front panel switches

DR		16	front panel display

MEMERR		16	memory error register

CCR		16	cache control register

MAINT		16	maintenance register

HITMISS		16	hit/miss register

CPUERR		16	CPU error register

PIRQ		16	programmed interrupt requests

FAC0H..FAC5H	32	FAC0..FAC5, high 32 bits

FAC0L..FAC5L	32	FAC0..FAC5, low 32 bits

FPS		16	floating point status

FEA		16	floating exception address

FEC		4	floating exception code

MMR0..3		16	memory management registers 0..3

{K/S/U}{I/D}{PAR/PDR}{0..7}

16	memory management registers

INT		32	interrupt pending flags

TRAP		18	trap pending flags

WAIT		0	wait state flag

WAIT_ENABLE	0	wait state enable flag

STOP_TRAPS	18	stop on trap flags

STOP_VECA	1	stop on read abort in trap or interrupt

STOP_SPA	1	stop on stack push abort in trap or interrupt

OLDPC		16	PC prior to last JMP, JMS, or interrupt

BREAK		16	breakpoint address (1 to disable)

WRU		8	interrupt character

5.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

CSR		16	control/status register

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		31	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

5.3 RK11/RK05

RK11 options include the ability to make units write enabled or write locked:

SET RKn LOCKED		set unit n write locked

SET RKn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The RK11 implements these registers:

name		size	comments

RKCS		16	control/status

RKDA		16	disk address

RKBA		16	memory address

RKWC		16	word count

RKDS		16	drive status

RKER		16	error status

INTQ		9	interrupt queue

DRVN		3	number of last selected drive

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

5.4 RX11/RX01

RX11 options include the ability to make units write enabled or write locked:

SET RXn LOCKED		set unit n write locked

SET RXn ENABLED		set unit n write enabled

The RX11 implements these registers:

name		size	comments

RXCS		12	status

RXDB		8	data buffer

RXES		8	error status

RXERR		8	error code

RXTA		8	current track

RXSA		8	current sector

STAPTR		3	controller state

BUFPTR		3	buffer pointer

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

TR		1	transfer ready flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

DONE		1	device done flag (CSR<5>)

CTIME		24	command completion time

STIME		24	seek time, per track

XTIME		24	transfer ready delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

5.5 RL11(V12)/RL01,RL02

RL11 options include the ability to set units write enabled or write locked,

and to set the drive size to RL01, RL02, or autosize:

SET RLn LOCKED		set unit n write locked

SET RLn ENABLED		set unit n write enabled

SET RLn RL01		set size to RL01

SET RLn RL02		set size to RL02

SET RLn AUTOSIZE	set size based on file size at attach

The size options can be used only when a unit is not attached to a file.

Units can also be REMOVEd or ADDed to the configuration.

The RL11 implements these registers:

name		size	comments

RLCS		16	control/status

RLDA		16	disk address

RLBA		16	memory address

RLBAE		6	memory address extension (RLV12)

RLMP..RLMP2	16	multipurpose register queue

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

5.6 Magnetic Tape (TM)

Magnetic tape options include the ability to make units write enabled or

or write locked.

SET TMn LOCKED		set unit n write locked

SET TMn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The magnetic tape controller implements these registers:

name		size	comments

MTS		16	status

MTC		16	command

MTCMA		16	memory address

MTBRC		16	byte/record count

INT		1	interrupt pending flag

ERR		1	error flag

DONE		1	device done flag

IE		1	interrupt enable flag

STOP_IOE	1	stop on I/O error

TIME		24	delay

UST0..7		16	unit status, units 0..n

POS0..7		31	position, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

5.7 Symbolic Display and Input

The PDP-11 simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as two character ASCII string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command

line switches:

' or -a			ASCII character

" or -c			two character ASCII string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard PDP-11 assembler syntax.  There are sixteen

instruction classes:

class		operands		examples		comments

no operands	none			HALT, RESET

3b literal	literal, 0 - 7		SPL

6b literal	literal, 0 - 077	MARK

8b literal	literal, 0 - 0377	EMT, TRAP

register	register		RTS

sop		specifier		SWAB, CLR, ASL

reg-sop		register, specifier	JSR, XOR, MUL

fop		flt specifier		ABSf, NEGf

ac-fop		flt reg, flt specifier	LDf, MULf

ac-sop		flt reg, specifier	LDEXP, STEXP

ac-moded sop	flt reg, specifier	LDCif, STCfi

dop		specifier, specifier	MOV, ADD, BIC

cond branch	address			BR, BCC, BNE

sob		register, address	SOB

cc clear	cc clear instructions	CLC, CLV, CLZ, CLN	combinable

cc set		cc set instructions	SEC, SEV, SEZ, SEN	combinable

For floating point opcodes, F and D variants, and I and L variants, may be

specified regardless of the state of FPS.

The syntax for specifiers is as follows:

syntax		specifier	displacement	comments

Rn		0n		-

Fn		0n		-		only in flt reg classes

(Rn)		1n		-

@(Rn)		7n		0		equivalent to @0(Rn)

(Rn)+		2n		-

@(Rn)+		3n              -

-(Rn)		4n              -

@-(Rn)		5n              -

{+/-}d(Rn)	6n		{+/-}d

@{+/-}d(Rn)	7n		{+/-}d

#n		27		n

@#n		37		n

.+/-n		67		+/-n - 4

@.+/-n		77		+/-n - 4

{+/-}n		67		{+/-}n - PC - 4	if on disk, 37 and n

@{+/-}n		77		{+/-}n - PC - 4	if on disk, invalid

6. Nova Features

The Nova simulator is configured as follows:

device		simulates

name(s)

CPU		Nova CPU with 32KW of memory

-		hardware multiply/divide

PTR,PTP		paper tape reader/punch

TTI,TTO		console terminal

LPT		line printer

CLK		real-time clock

DK		head-per-track disk controller

DP		moving head disk controller with four drives

MT		magnetic tape controller with eight drives

The Nova simulator implements these unique stop conditions:

- reference to undefined I/O device, and STOP_DEV is set

- more than INDMAX indirect addresses are detected during

an interrupt

- more than INDMAX indirect addresses are detected during

memory reference address decoding

6.1 CPU

The only CPU options are the presense of the multiply/divide instructions

and the size of main memory.

SET CPU MDV		enable multiply/divide

SET CPU NOMDV		disable multiply/divide

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 32K.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		15	program counter

AC0..AC3	16	accumulators 0..3

C		1	carry

SR		16	front panel switches

PI		16	priority interrupt mask

ION		1	interrupt enable

ION_DELAY	1	interrupt enable delay for ION

PWR		1	power fail interrupt

INT		15	interrupt pending flags

BUSY		15	device busy flags

DONE		15	device done flags

DISABLE		15	device interrupt disable flags

STOP_DEV	1	stop on undefined IOT

INDMAX		15	maximum number of nested indirects

OLDPC		15	PC prior to last JMP, JMS, or interrupt

BREAK		16	breakpoint address (177777 to disable)

WRU		8	interrupt character

6.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The terminal reads and writes to the controlling console port.  Terminal

options include the ability to set limited Dasher compatibility mode or

normal mode:

SET TTI ANSI		normal mode

SET TTI DASHER		Dasher mode

SET TTO ANSI		normal mode

SET TTO DASHER		Dasher mode

Setting either TTI or TTO changes both devices.  In Dasher mode, carriage

return is changed to newline on input, and ^X is changed to backspace.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		31	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

6.3 Fixed Head Disk (DK)

The fixed head disk controller implements these registers:

name		size	comments

STAT		16	status

DA		16	disk address

MA		16	memory address

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	device disable flag

INT		1	interrupt pending flag

WLK		8	write lock switches

TIME		24	rotational delay, per sector

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

Fixed head disk data files are buffered in memory; therefore, end of file

and OS I/O errors cannot occur.

6.4 Moving Head Disk (DP)

Moving head disk options include the ability to make units write enabled or

write locked, and to select the type of drive:

SET DPn LOCKED		set unit n write locked

SET DPn ENABLED		set unit n write enabled

SET DPn FLOPPY		set unit n to floppy disk

SET DPn D31		set unit n to Diablo 31

SET DPn D44		set unit n to Diablo 44

SET DPn C111		set unit n to Century 111

SET DPn C114		set unit n to Century 114

Units can also be REMOVEd or ADDed to the configuration.

All drives have 256 16b words per sector.  The other disk parameters are:

drive cylinders surfaces sectors size (MW) 	DG models

floppy	77	1	8	  .158		6038

D31	203	2	12	 1.247		4047, 4237, 4238

D44	408	4	12	 5.014		4234, 6045

C111	203	10	6	 3.118		4048

C114	203	20	12	12.472		4057, 2314

The moving head disk controller implements these registers:

name		size	comments

FCCY		16	flags, command, cylinder

USSC		16	unit, surface, sector, count

STAT		16	status

MA		16	memory address

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

Error handling is as follows:

error		processed as

not attached	disk not ready

end of file	assume rest of disk is zero

OS I/O error	report error and stop

6.5 Magnetic Tape (MT)

Magnetic tape options include the ability to make units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The magnetic tape controller implements these registers:

name		size	comments

CU		16	command, unit

MA		16	memory address

WC		16	word count

STA1		16	status word 1

STA2		16	status word 2

EP		1	extended polling mode (not supported)

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

CTIME		24	controller delay

RTIME		24	record delay

UST0..7		32	unit status, units 0..n

POS0..7		31	position, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

6.6 Symbolic Display and Input

The Nova simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as two character ASCII string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command

line switches:

' or -a			ASCII character

" or -c			two character ASCII string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard Nova assembler syntax.  There are three

instruction classes: memory reference, IOT, and operate.

Memory reference instructions have the format

memref {ac,}{@}address{,index}

LDA and STA require an initial register; ISZ, DSZ, JSR, and JMP do not.

The syntax for addresses and indices is as follows:

syntax		mode	displacement	comments

0 <= n < 0400	0	n

{+/-}n >= 0400	1	{+/-}n - PC	must be in range [-200, 177]

invalid on disk

.+/-n		1	{+/-}n		must be in range [-200, 177]

{+/-}n,2	2	{+/-}n		must be in range [-200, 177]

{+/-}n,3	3	{+/-}n		must be in range [-200, 177]

IOT instructions have one of four formats

syntax			example

iot			HALT

iot reg			INTA

iot device		SKPDN

iot reg,device		DOAS

Devices may be specified as mnemonics or as numbers in the range 0 - 077.

Operate instructions have the format

opcode{#} reg,reg{,skip}

In all Nova instructions, blanks may be substituted for commas as field

delimiters.

7. 18b PDP Features

The four 18b PDP's (PDP-4, PDP-7, PDP-9, PDP-15) are very similar and

are configured as follows:

system	device		simulates

name(s)

PDP-4	CPU		PDP-4 CPU with 8KW of memory

PTR,PTP		integral paper tape/Type 75 punch

TTI,TTO		KSR28 console terminal (Baudot code)

LPT		Type 62 line printer (Hollerith code)

CLK		integral real-time clock

PDP-7	CPU		PDP-7 CPU with 32KW of memory

-		Type 177 extended arithmetic element (EAE)

-		Type 148 memory extension

PTR,PTP		Type 444 paper tape reader/Type 75 punch

TTI,TTO		KSR 33 console terminal

LPT		Type 647 line printer

CLK		integral real-time clock

DRM		Type 24 serial drum

PDP-9	CPU		PDP-9 CPU with 32KW of memory

-		KE09A extended arithmetic element (EAE)

-		KG09B memory extension

-		KP09A power detection

-		KX09A memory protection

PTR,PTP		PC09A paper tape reader/punch

TTI,TTO		KSR 33 console terminal

LPT		Type 647E line printer

CLK		integral real-time clock

RF		RF09/RS09 fixed-head disk

MT		TC59/TU10 magnetic tape

PDP-15	CPU		PDP-15 CPU with 32KW of memory

-		KE15 extended arithmetic element (EAE)

-		KF15 power detection

-		KM15 memory protection

PTR,PTP		PC15 paper tape reader/punch

TTI,TTO		KSR 35 console terminal

LPT		LP15 line printer

CLK		integral real-time clock

RP		RP15/RP02 disk pack

RF		RF15/RS09 fixed-head disk

MT		TC59/TU10 magnetic tape

The 18b PDP simulators implement one unique stop condition: if an undefined

instruction (unimplemented OPR) is decoded, and register STOP_INST is set,

the simulator halts.

7.1 CPU

The only CPU options are the presense of the EAE and the size of main memory.

SET CPU EAE		enable EAE

SET CPU NOEAE		disable EAE

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

SET CPU 48K		set memory size = 48K

SET CPU 64K		set memory size = 64K

SET CPU 80K		set memory size = 80K

SET CPU 96K		set memory size = 96K

SET CPU 112K		set memory size = 112K

SET CPU 128K		set memory size = 128K

Memory sizes greater than 8K are only available on the PDP-7, PDP-9, and

PDP-15; memory sizes greater than 32KW are only available on the PDP-15.

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 8K for the PDP-4, 32K

for the PDP-7 and PDP-9, and 128K for the PDP-15.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

system	name		size	comments

all	PC		addr	program counter

7,9	PC		15	program counter

15	PC		17	program counter

all	AC		18	accumulator

7,9,15	MQ		18	multiplier-quotient

7,9,15	SC		6	shift counter

7,9,15	EAE_AC_SIGN	1	EAE AC sign

all	L		1	link

7,9	EXTM		1	extend mode

15	BANKM		1	bank mode

7	TRAPM		1	trap mode

9,15	USMD		1	user mode

9,15	USMDBUF		1	user mode buffer

9,15	BR		addr	memory protection bounds

7,9,15	TRAPP		1	trap pending

9,15	NEXM		1	non-existent memory violation

9,15	PRVN		1	privilege violation

7,9	EMIRP		1	EMIR instruction pending

9,15	RESTP		1	DBR or RES instruction pending

15	XR		18	index register

15	LR		18	limit register

all	SR		18	front panel switches

all	INT		32	interrupt requests

all	IORS		18	IORS register

all	ION		1	interrupt enable

all	ION_DELAY	2	interrupt enable delay

all	OLDPC		addr	PC prior to last transfer

all	STOP_INST	1	stop on undefined instruction

all	BREAK		18	breakpoint address (177777 to disable)

all	WRU		8	interrupt character

"addr" signifies the address width of the system (13b for the PDP-4, 15b for

the PDP-7 and PDP-9, 17b for the PDP-15).

7.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

INT		1	interrupt pending flag

DONE		1	device done flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		31	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

7.3 RP15/RP02

RP15 options include the ability to make units write enabled or write locked:

SET RPn LOCKED		set unit n write locked

SET RPn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The RP15 implements these registers:

name		size	comments

STA		18	status A

STB		18	status B

DA		18	disk address

MA		18	current memory address

WC		18	word count

INT		1	interrupt pending flag

BUSY		1	control busy flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

7.4 DRM

The drum has no options.  It implements these registers:

name		size	comments

DA		9	drum address (sector number)

MA		15	current memory address

INT		1	interrupt pending flag

DONE		1	device done flag

ERR		1	error flag

WLK		32	write lock switches

TIME		24	rotational latency, per word

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

Drum data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

7.5 RF09/RF15/RS09

The RF09/RF15 implements these registers:

name		size	comments

STA		18	status

DA		21	current disk address

MA		18	memory address (in memory)

WC		18	word count (in memory)

BUF		18	data buffer (diagnostic only)

INT		1	interrupt pending flag

WLK0..7		16	write lock switches for disks 0..7

TIME		24	rotational delay, per word

BURST		1	burst flag

STOP_IOE	1	stop on I/O error

The RF09/RF15 is a three-cycle data break device.  If BURST = 0, word

transfers are scheduled individually; if BURST = 1, the entire transfer

occurs in a single data break.

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RF15/RF09 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

7.6 Magnetic Tape (MT)

Magnetic tape options include the ability to make units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The magnetic tape controller implements these registers:

name		size	comments

CMD		18	command

STA		18	main status

MA		18	memory address (in memory)

WC		18	word count (in memory)

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

TIME		24	record delay

UST0..7		24	unit status, units 0..n

POS0..7		31	position, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

7.7 Symbolic Display and Input

The 18b PDP simulators implement symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as (sixbit) character string

-m			display instruction mnemonics

The PDP-15 also recognizes an additional switch:

-p			display as packed ASCII (five 7b ASCII

characters in two 18b words)

Input parsing is controlled by the first character typed in or by command

line switches:

' or -a			ASCII character

" or -c			three character sixbit string

alphabetic		instruction mnemonic

numeric			octal number

The PDP-15 also recognizes an additional input mode:

# or -p			five character packed ASCII string in

two 18b words

Instruction input uses standard 18b PDP assembler syntax.  There are six

instruction classes: memory reference, EAE, index (PDP-15 only), IOT,

operate, and LAW.

Memory reference instructions have the format

memref {I/@} address{,X}

where I (PDP-4, PDP-7, PDP-9) /@ (PDP-15) signifies indirect reference,

and X signifies indexing (PDP-15 in page mode only).  The address is an

octal number in the range 0 - 017777 (PDP-4, PDP-7, PDP-9, and PDP-15 in

bank mode) or 0 - 07777 (PDP-15 in page mode).

IOT instructions consist of single mnemonics, eg, KRB, TLS.  IOT instructions

may be or'd together

iot iot iot...

IOT's may also include the number 10, signifying clear the accumulator

iot 10

The simulator does not check the legality of IOT combinations.  IOT's for

which there is no opcode may be specified as IOT n, where n is an octal

number in the range 0 - 07777.

EAE instructions have the format

eae {+/- shift count}

EAE instructions may be or'd together

eae eae eae...

The simulator does not check the legaility of EAE combinations.  EAE's for

which there is no opcode may be specified as EAE n, where n is an octal

number in the range 0 - 037777.

Index instructions (PDP-15 only) have the format

index {immediate}

The immediate, if allowed, must be in the range of -0400 to +0377.

Operate instructions have the format

opr opr opr...

The simulator does not check the legality of the proposed combination.  The

operands for MUY and DVI must be deposited explicitly.

Finally, the LAW instruction has the format

LAW immediate

where immediate is in the range of 0 to 017777.

8. IBM 1401 Features

The IBM 1401 simulator is configured as follows:

device		simulates

name(s)

CPU		IBM 1401 CPU with 16K of memory

CDR,CDP		IBM 1402 card reader/punch

LPT		IBM 1403 line printer

INQ		IBM 1407 inquiry terminal

MT		IBM 729 7-track magnetic tape controller with six drives

The IBM 1401 simulator implements many unique stop conditions.  On almost

any kind of error the simulator stops:

unimplemented opcode

reference to non-existent memory

reference to non-existent device

no word mark under opcode

invalid A address

invalid B address

invalid instruction length

invalid modifier character

invalid branch address

invalid magtape unit number

invalid magtape record length

write to locked magtape drive

skip to unpunched carriage control tape channel

card reader hopper empty

address register wrap-around

single character A field in MCE

single character B field in MCE

hanging $ in MCE with EPE enabled

I/O check with I/O stop switch set

8.1 CPU

The CPU options include a number of special features and the size of main

memory.  Note that the Modify Address special feature is always included

when memory size is greater than 4K.

SET CPU XSA		enable advanced programming special feature

SET CPU NOXSA		disable advanced programming

SET CPU HLE		enable high/low/equal special feature

SET CPU NOHLE		disable high/low/equal

SET CPU BBE		enable branch on bit equal special feature

SET CPU NOBBE		disable branch on bit equal

SET CPU MR		enable move record special feature

SET CPU NOMR		disable move record

SET CPU EPE		enable extended print edit special feature

SET CPU NOEPE		disable extended print edit

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initially, memory size is 16K, and all special

features are enabled.

Memory is implemented as 7 bit BCD characters, as follows:

6	5 	4	3	2	1	0

word	B bit	A bit	8	4	2	1

mark	<-- zone -->	<-------- digit -------->

In BCD, the decimal digits 0-9 are (octal) values 012, 001, 002, 003, 004,

005, 006, 007, 010, 011, respectively.  Signs are encoded in the zone bits,

with 00, 01, and 11 being positive, and 10 being negative.

CPU registers include the visible state of the processor.  The 1401 has no

interrupt system.

name		size	comments

IS		14	instruction storage address register (PC)

AS		14	A storage address register

BS		14	B storage address register

ASERR		1	AS invalid flag

BSERR		1	BS invalid flag

SSA		1	sense switch A

SSB		1	sense switch B

SSC		1	sense switch C

SSD		1	sense switch D

SSE		1	sense switch E

SSF		1	sense switch F

SSG		1	sense switch G

EQU		1	equal compare indicator

UNEQ		1	unequal compare indicator

HIGH		1	high compare indicator

LOW		1	low compare indicator

OVF		1	overflow indicator

IOCHK		1	I/O check switch

PRCHK		1	process check switch

OLDIS		1	IS prior to last branch

BREAK		17	breakpoint address (1000000 to disable)

WRU		8	interrupt character

8.2 Card Reader/Punch

The IBM 1402 card/reader punch is simulated as three independent devices:

the card reader (CDR), the card punch (CDP), and the reader and punch

stackers (STKR).  STRK units 0, 1, 2, and 4 correspond to the reader

normal stacker, reader stacker 1, shared stacker 2/8, and punch stacker

4, respectively.

The card reader reads data from disk files, while the punch and stackers

write data to disk files.  Cards are simulated as ASCII text lines with

terminating newlines; column binary is not supported.  For each unit,

the POS register specifies the number of the next data item to be read or

written.  Thus, by changing POS, the user can backspace or advance these

devices.

The reader/punch registers are:

device	name		size	comments

CDR	LAST		1	last card indicator

ERR		1	error indicator

S1		1	stacker 1 select flag

S2		1	stacker 2 select flag

POS		31	position

TIME		24	delay window for stacker select

CDP	ERR		1	error indicator

S4		1	stacker 4 select flag

S8		1	stacker 8 select flag

STKR	POS0		31	position, normal reader stack

POS1		31	position, reader stacker 1

POS2		31	position, shared stacker 2/8

POS4		31	position, punch stacker 4

Error handling is as follows:

device		error		processed as

reader		end of file	if SSA set, set LAST indicator

on next Read, report error and stop

reader,punch	not attached	report error and stop

OS I/O error	print error message

if IOCHK set, report error and stop

otherwise, set ERR indicator

stacker		not attached	ignored

OS I/O error	print error message

if IOCHK set, report error and stop

8.3 Line Printer

The IBM 1403 line printer (LPT) writes its data, converted to ASCII, to

a disk file.  The line printer supports three different print character

sets or "chains":

SET LPT PCF			full 64 character chain

SET LPT PCA			48 character business chain

SET LPT PCH			48 character FORTRAN chain

In addition, the line printer can be programmed with a carriage control

tape.  The LOAD command loads a new carriage control tape:

LOAD <file>			load carriage control tape file

The format of a carriage control tape consists of multiple lines.  Each

line contains an optional repeat count, enclosed in parentheses, optionally

followed by a series of column numbers separated by commas.  Column numbers

must be between 1 and 12; a column number of zero denotes top of form.  The

following are all legal carriage control specifications:

<blank line>			no punch

(5)				5 lines with no punches

1,5,7,8				columns 1, 5, 7, 8 punched

(10)2				10 lines with column 2 punched

1,0				column 1 punched; top of form

The default form is 66 lines long, with column 1 and the top of form mark

on line 1, and the rest blank.

The line printer registers are:

name		size	comments

LINES		8	number of newlines after next print

LFLAG		1	carriage control flag (1 = skip, 0 = space)

CCTP		8	carriage control tape pointer

CCTL		8	carriage control tape length (read only)

ERR		1	error indicator

POS		31	position

Error handling is as follows:

error		processed as

not attached	report error and stop

OS I/O error	print error message

if IOCHK set, report error and stop

otherwise, set ERR indicator

8.4 Inquiry Terminal

The IBM 1407 inquiry terminal (INQ) is a half-duplex console.  It polls

the controlling keyboard of the simulator periodically for inquiry requests.

The inquiry terminal registers are:

name		size	comments

INQC		7	inquiry request character (initially ESC)

INR		1	inquiry request indicator

INC		1	inquiry cleared indicator

TIME		24	polling interval

When the 1401 CPU requests input from the keyboard, the message [Enter]

is printed out, followed by a new line.  The CPU hangs waiting for input

until either the return/enter key is pressed, or the inquiry request

character is typed in.  The latter cancels the type-in and sets INC.

The inquiry terminal has no errors.

8.5 Magnetic Tape (MT)

The magnetic tape controller supports six drives, numbered 1 through 6.

Magnetic tape options include the ability to make units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

Units can also be REMOVEd or ADDed to the configuration.

The magnetic tape controller implements these registers:

name		size	comments

END		1	end of file indicator

ERR		1	error indicator

PAR		1	parity error indicator

POS1..6		31	position, drives 1..6

Error handling is as follows:

error		processed as

not attached	report error and stop

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	print error message

if IOCHK set, report error and stop

otherwise, set ERR indicator

8.6 Symbolic Display and Input

The IBM 1401 simulator implements symbolic display and input.  Display is

controlled by command line switches:

-c			display as single character

(BCD for CPU and MT, ASCII for others)

-s			display as wordmark terminated BCD string

(CPU only)

-m			display instruction mnemonics

(CPU only)

In a CPU character display, word marks are denoted by ~.

Input parsing is controlled by the first character typed in or by command

line switches:

' or " or -c or -s	characters (BCD for CPU and MT, ASCII

for others)

alphabetic		instruction mnemonic

numeric			octal number

Instruction input is free format, with spaces separating fields.  There

are six instruction formats: 1, 2, 4, 5, 7, and 8 characters:

1 character		opcode

2 character		opcode 'modifier

4 character		opcode address

5 character		opcode address 'modifier

7 character		opcode address address

8 character		opcode address address 'modifier

Addresses are always decimal, except for special I/O addresses in the A

field, which may be specified as %xy, where x denotes the device and y

the unit number.

For the CPU, string input may encompass multiple characters.  A word mark

is denoted by ~ and must precede the character to be marked.  All other

devices can only accept single character input, without word marks.

Appendix 1: Notes On File Representations

1. Disks

Hard disks are represented as unstructured binary files of 16b data items.

Thus, simulated hard disks are endian dependent.  To share a disk file

between versions of the simulator running on big and little endian systems,

each 16b word must be byte swapped prior to use.

PDP-8 and PDP-11 floppy disks are represented as unstructured binary files

of 8b data items.  They are nearly identical to the floppy disk images for

Doug Jones' PDP-8 simulator but lack the initial 256 byte header.  A utility

for converting between the two formats is easily written.

2. Magnetic Tapes

Magnetic tapes are represented as unstructured binary files of 8b data

items.  Each record consists of a 16b byte count n, in little endian

format (low 8b first, then high 8b), followed by n 8b bytes of data.

If the byte count is odd, the record is padded to even length; the pad

byte is undefined.

Magnetic tapes are endian independent and consistent across simulator

families.  A magtape produced by the Nova simulator will appear to

have its 16b words byte swapped if read by the PDP-11 simulator.

3. Line Printers

Line printer output is represented by an ASCII file of lines separated by

the newline character.  Overprinting is represented by a line ending in

return rather than newline.

Appendix 2: Sample Software

1. PDP-8 FOCAL69

FOCAL69 is an interactive program for technical computations.  It can

execute both immediate commands and stored programs (like BASIC).  FOCAL69

is provided as a binary loader format paper-tape image.  To load and

execute FOCAL69:

sim> load focal69.bin

sim> run 200

*type 2+2

=    4.000*

2. PDP-8 OS/8

OS/8 is the PDP-8's mass storage-based operating system.  It provides a

program development and execution environment for assembler, BASIC, and

FORTRAN programs.  OS/8 is provided under license, as is, without fee, by

Digital Equipment Corporation, for non-commercial use only.  Please read

the enclosed license agreement for full terms and conditions.  This license

agreement must be reproduced with any copy of the OS/8 disk images.  My

thanks to Doug Jones of the University of Iowa, who provided the disk

images, and to Digital Equipment Corporation, which provided the license.

To load and run OS/8:

sim> att rx0 os8sys_rx.dsk

sim> att rx1 os8f4_rx.dsk

sim> boot rx0

.DA dd-mmm-yy

.

Note that OS/8 only recognizes upper case characters.  The first disk

(drive 0) is the system disk; it also includes BASIC.  The second disk

(drive 1) includes FORTRAN.

3. PDP-11 UNIX V5, V6, V7

UNIX was first developed on the PDP-7; its first widespread usage was on

the PDP-11.  UNIX provides a program development and execution environment

for assembler and C programs.  UNIX V5, V7, V7 for the PDP-11 is provided

under license, as is, without fee, by Santa Cruz Organization (SCO), for

non-commercial use only.  Please read the enclosed license agreement for

full terms and conditions.  This license must be reproduced with any copy

of the UNIX V5, V6, V7 disk images.  My thanks to PUPS, the PDP-11 UNIX

Preservation Society of Australia, which provided the disk images, and to

SCO, which provided the license.

3.1 UNIX V5

Unix V5 is contained on a single RK05 disk image.  To boot UNIX:

sim> set cpu 18b

sim> att rk0 unix_v5_rk.dsk

sim> boot rk

@unix

login: root

#ls -l

3.2 UNIX V6

Unix V6 is contained on three RK05 disk images.  To boot Unix:

sim> set cpu 18b

sim> att rk0 unix0_v6_rk.dsk

sim> att rk1 unix1_v6_rk.dsk

sim> att rk3 unix3_v6_rk.dsk

sim> boot rk0

@unix

login: root

# ls -l

3.3 UNIX V7

Unix V7 is contained on a single RL02 disk image.  To boot Unix:

sim> set cpu 18b

sim> set rl0 RL02

sim> att rl0 unix_v7_rl.dsk

sim> boot rl0

@unix

login: root

password: pdp

# ls -l

A smaller image is contained on a single RK05 disk image.  To boot Unix:

sim> set cpu 18b

sim> att rk0 unix_v7_rk.dsk

sim> boot rk0

@rkunix

login: root

password: pdp

# ls -l

4. 1401 Single Card "Koans"

One of the art forms for the IBM 1401 was packing useful programs into a

single punched card.  Three samples are included:

i1401_ctolp.cd		prints a card deck on the line printer

i1401_ctopu.cd		copies a card deck to the card punch

i1401_hello.cd		prints "HELLO WORLD" on the line printer and stops

To use the reproduction cards, simply insert them at the beginning of a

text file, terminated by newline.  Attach the modified file to the card

reader, attach a blank file to the output device, and boot the card reader.

Appendix 3: Debug Status

The debug status of each simulated CPU and device is as follows:

system	PDP-8	PDP-11	Nova	18b PDP's	IBM 1401

device

CPU		y	y	y	h		h

FPU		-	h	-	-		-

console		y	y	y	h		h

paper tape	y	y	y	h		-

card reader	-	-	-	-		h

line printer	y	y	y	h		h

clock		y	y	y	h		-

hard disk	y	y	y	h		-

fixed disk	h	-	h	h		-

floppy disk	y	y	y	h		-

mag tape	h	h	y	h		h

legend:		y = runs operating system

d = runs diagnostics

h = runs hand-generated test cases

n = untested

- = not applicable

Revision History (since Rev 1.1)

Rev 2.2d, Dec, 96 Added ADD/REMOVE commands

Added unit enable/disable support to device simulators

Added features for IBM 1401 project

Added switch recognition for symbolic input

Fixed bug in variable length IEXAMINE

Fixed LCD bug in RX8E

Initial changes for Win32

Added IBM 1401

Rev 2.2b, Apr, 96 Added PDP-11 dynamic memory size support

Rev 2.2a, Feb, 96 New endian independent magtape format

Rev 2.2 Jan, 96	Added register buffers for save/restore

Added 18b PDP's

Guaranteed TTI, CLK times are non-zero

Fixed breakpoint/RUN interaction bug

Fixed magnetic tape backspace to EOF bug

Fixed ISZ/DCA inversion in PDP-8 symbol table

Fixed sixbit conversion in PDP-8 examine/deposit

Fixed origin increment bug in PDP-11 binary loader

Fixed GCC longjmp optimization bug in PDP-11 CPU

Fixed unit number calculation bug in SCP and in

Nova, PDP-11, 18b PDP moving head disks

Rev 2.1	Dec, 95	Fixed PTR bug (setting done on EOF) in PDP-8, Nova

Fixed RX bug (setting error on INIT if drive 1 is

not attached) in PDP-8, PDP-11

Fixed RF treatment of photocell flag in PDP-8

Fixed autosize bug (always chose smallest disk if new

file) in PDP-11, Nova

Fixed not attached bug (reported as not attachable) in

most mass storage devices

Fixed Nova boot ROMs

Fixed bug in RESTORE (didn't requeue if delay = 0)

Fixed bug in RESTORE (clobbered device position)

Declared static constant arrays as static const

Added PDP-8, Nova magnetic tape simulators

Added Dasher mode to Nova terminal simulator

Added LINUX support

Rev 2.0	May, 95	Added symbolic assembly/disassembly